CRITICAL

MASTER THE ATOM

PHASE 2: REACTOR CORE

with Automated Auxiliaries

Unity 6.3 Implementation | Duration: 3-4 weeks

Gate: Criticality & Power Response Validated

# 1. Phase Overview

Phase 2 builds the reactor core using Phase 1's validated physics. All supporting systems run in AUTO mode with perfect boundary conditions.

## 1.1 Entry Criteria

✓ Phase 1 Gate PASSED (all 45+ physics tests passing)

✓ No modifications to Phase 1 code

## 1.2 System Status

| System | Mode | Control |
| --- | --- | --- |
| Reactor Core | MANUAL | Rods, Power |
| Control Rods | MANUAL | IN/OUT/TRIP |
| Pressurizer | AUTO | Holds 2235 psig |
| Steam Generators | AUTO | Holds 1000 psia |
| CVCS | AUTO | Boron setpoint only |
| RCPs/Turbine | AUTO | 100% flow/load |

# 2. Core Implementation

## 2.1 ReactorState.cs

public struct ReactorState {

public float neutronPower;      // 0-1+ fraction

public float thermalPower;      // 0-1 fraction (lagged)

public float fuelTemperature;   // °F

public float reactivity;        // pcm

public float[] precursors;      // 6-group

public float rodPosition;       // 0-228 steps

public float Tcold, Thot, Tavg; // °F

public float pressure;          // psig

public float boronConcentration; // ppm

}

## 2.2 SimulationManager.cs Loop

void SimulationStep(float dt) {

rods.Update(ref state, dt);

state.reactivity = reactor.CalculateReactivity(state);

reactor.UpdatePower(ref state, dt);

reactor.UpdateTemperatures(ref state, dt);

// Auto systems maintain setpoints

pressurizer.Update(ref state); // holds 2235

cvcs.Update(ref state);        // instant boron

}

## 2.3 Reactivity Calculation

public float CalculateReactivity(ReactorState state) {

float rho = 0f;

rho += RodWorthCurve(state.rodPosition);     // 0-1000 pcm

rho += ReactorKinetics.DopplerReactivity(...);

rho += ReactorKinetics.ModeratorReactivity(...);

rho += ReactorKinetics.BoronReactivity(...);

return rho;

}

## 2.4 Control Rod Worth Curve

// S-curve: low worth at top/bottom, high in middle

float RodWorthCurve(float position) {

float x = position / 228f;  // 0-1 normalized

return 1000f * (3f * x * x - 2f * x * x * x);

}

# 3. Test Scenarios

## 3.1 Reactor Startup

| Step | Action | Expected |
| --- | --- | --- |
| 1 | Initial: Rods=0, Boron=2000 | Power~0, ρ very negative |
| 2 | Reduce boron to 1300 ppm | ρ increases toward 0 |
| 3 | Withdraw rods slowly | Watch ρ approach 0 |
| 4 | Achieve criticality | ρ=0, SUR=0, power steady |
| 5 | Continue withdrawal | SUR positive, power rises |
| 6 | Reach 50% power | Tavg=567°F per program |

## 3.2 Reactor Trip

| Step | Action | Expected |
| --- | --- | --- |
| 1 | Start: 100% power | All parameters normal |
| 2 | Press SPACE (trip) | Rods insert rapidly |
| 3 | Rods at bottom (2s) | ρ ~ -5000 pcm |
| 4 | Power decay | <5% in 3 seconds |

# 4. Phase 2 Test Suite

## 4.1 Unit Tests

| ID | Test Case | Pass |
| --- | --- | --- |
| RC-001 | Rod worth at 0 steps = 0 pcm | □ |
| RC-002 | Rod worth at 228 steps = ~1000 pcm | □ |
| RC-003 | Tavg program: 547°F at 0%, 588°F at 100% | □ |
| RC-004 | Delta-T = 61°F at 100% power | □ |
| RC-005 | Fuel temp = Tavg + 400°F at 100% | □ |
| RC-006 | Thermal power lags neutron (τ=5s) | □ |
| RC-007 | Trip inserts rods in 2 seconds | □ |
| RC-008 | Auto pressurizer holds 2235±1 psig | □ |

## 4.2 Integration Tests

| ID | Test Case | Pass |
| --- | --- | --- |
| INT-101 | Achieve criticality (ρ = 0 ± 5 pcm) | □ |
| INT-102 | Power responds to rod motion | □ |
| INT-103 | Doppler stabilizes power (no runaway) | □ |
| INT-104 | Trip reduces power <5% in 3 seconds | □ |
| INT-105 | Tavg follows program through power change | □ |
| INT-106 | Complete startup: subcritical → 100% | □ |
| INT-107 | 100% steady state: all params nominal | □ |
| INT-108 | +100 pcm step: power rises then stabilizes | □ |

## 4.3 Acceptance Criteria

| Parameter | Expected @100% | Tolerance |
| --- | --- | --- |
| Neutron Power | 100% | ±0.5% |
| Tavg | 588°F | ±2°F |
| Thot | 619°F | ±3°F |
| Tcold | 558°F | ±3°F |
| ΔT | 61°F | ±2°F |
| Equilibrium ρ | 0 pcm | ±5 pcm |

# 5. Phase 2 Exit Gate

ALL criteria must be met before proceeding to Phase 3:

## 5.1 Gate Checklist

| Requirement | Status | Date |
| --- | --- | --- |
| Reactor achieves criticality via rod withdrawal | □ PASS | ______ |
| Power responds correctly to rod motion | □ PASS | ______ |
| Negative feedback prevents power runaway | □ PASS | ______ |
| Reactor trip reduces power <5% in 3s | □ PASS | ______ |
| Steady-state temps match Tavg program | □ PASS | ______ |
| All 8 unit tests pass | □ PASS | ______ |
| All 8 integration tests pass | □ PASS | ______ |
| Code review approved | □ PASS | ______ |
| PHASE 2 GATE APPROVED | □ YES | ______ |

Phase 2 Gate Passed → Proceed to Phase 3: Pressurizer System