CRITICAL

MASTER THE ATOM

PHASE 3: PRESSURIZER

Auto → Manual Conversion

Unity 6.3 Implementation | Duration: 2-3 weeks

Gate: Pressure Response to Thermal Transients Validated

# 1. Phase Overview

Phase 3 replaces AUTO pressurizer with physics-based model. First system that 'fights back' with real dynamics.

## 1.1 Entry Criteria

✓ Phase 2 Gate PASSED

✓ Reactor achieves criticality and responds to rod motion

## 1.2 System Status After Phase 3

| System | Mode | Notes |
| --- | --- | --- |
| Reactor Core | MANUAL | From Phase 2 |
| Pressurizer | MANUAL | THIS PHASE |
| Steam Generators | AUTO | Holds 1000 psia |
| CVCS | AUTO | Instant boron |
| RCPs/Turbine | AUTO | 100% flow/load |

# 2. Pressurizer Physics

## 2.1 State Variables

public struct PressurizerState {

public float pressure;        // psig

public float liquidVolume;    // ft³

public float steamVolume;     // ft³

public float level;           // percent

public float heaterPower;     // kW (0-1800)

public float sprayFlow;       // gpm (0-900)

}

## 2.2 Pressure Dynamics

Pressure changes from: 1) Surge (thermal expansion), 2) Heaters (boil water), 3) Spray (condense steam)

// Polytropic compression: P2 = P1 * (V1/V2)^1.3

float newPressure = SteamThermodynamics.PolytropicPressure(

state.pressure, oldSteamVol, newSteamVol);

## 2.3 Control Setpoints

| Setpoint | Pressure | Action | Purpose |
| --- | --- | --- | --- |
| Spray On | >2260 psig | Proportional spray | Reduce pressure |
| Normal | 2235 psig | Proportional heaters | Maintain setpoint |
| Heaters On | <2210 psig | All 1800 kW | Restore pressure |
| PORV Open | >2335 psig | Relief valve lifts | Prevent safety lift |
| Safety Valve | >2485 psig | Code safety opens | Ultimate protection |

# 3. Test Scenarios

## 3.1 Power Increase Transient

Start: 50% power → Action: Raise to 75% → Expected: Pressure spike ~20 psi, spray activates, returns to 2235

## 3.2 Power Decrease Transient

Start: 100% power → Action: Reduce to 75% → Expected: Pressure drop ~20 psi, heaters activate, returns to 2235

## 3.3 Loss of Heaters

Start: Normal → Action: Trip heaters → Expected: Slow pressure decay, operator must respond

# 4. Phase 3 Test Suite

## 4.1 Unit Tests

| ID | Test Case | Pass |
| --- | --- | --- |
| PZ-001 | Surge IN compresses steam → pressure rises | □ |
| PZ-002 | Surge OUT expands steam → pressure drops | □ |
| PZ-003 | Heaters create steam → pressure rises | □ |
| PZ-004 | Spray condenses steam → pressure drops | □ |
| PZ-005 | Level = liquidVolume / 1800 × 100 | □ |
| PZ-006 | PORV opens at 2335 psig | □ |
| PZ-007 | Heater power limited to 1800 kW | □ |
| PZ-008 | Spray flow limited to 900 gpm | □ |

## 4.2 Integration Tests

| ID | Test Case | Pass |
| --- | --- | --- |
| INT-201 | 10% power increase → pressure spike ~20 psi | □ |
| INT-202 | 10% power decrease → pressure drop ~20 psi | □ |
| INT-203 | Pressure returns to 2235±10 within 60s | □ |
| INT-204 | 100% load rejection → no PORV lift (<2335) | □ |
| INT-205 | Level tracks programmed value (25-60%) | □ |
| INT-206 | Reactor trip → pressure spike then recovery | □ |
| INT-207 | Steady state pressure drift < 5 psi/hour | □ |
| INT-208 | Steam bubble maintained (no solid PZR) | □ |

# 5. Phase 3 Exit Gate

| Requirement | Status | Date |
| --- | --- | --- |
| Pressure responds to thermal transients (±20 psi) | □ PASS | ______ |
| Heaters/spray maintain pressure within band | □ PASS | ______ |
| Level tracks programmed value | □ PASS | ______ |
| PORV opens at setpoint | □ PASS | ______ |
| Steam bubble maintained | □ PASS | ______ |
| All 8 unit tests pass | □ PASS | ______ |
| All 8 integration tests pass | □ PASS | ______ |
| PHASE 3 GATE APPROVED | □ YES | ______ |

Phase 3 Gate Passed → Proceed to Phase 4: CVCS & RCPs