CRITICAL

MASTER THE ATOM

PHASE 4: CVCS & RCPs

Inventory Control & Coolant Flow

Unity 6.3 Implementation | Duration: 2-3 weeks

Gate: Boron Transport & Pump Coastdown Validated

# 1. Phase Overview

Phase 4 adds CVCS (boron control with transport delay) and RCPs (flow dynamics with coastdown).

## 1.1 Entry Criteria

✓ Phase 3 Gate PASSED

✓ Pressurizer responds to thermal transients

## 1.2 System Status After Phase 4

| System | Mode | Notes |
| --- | --- | --- |
| Reactor Core | MANUAL | From Phase 2 |
| Pressurizer | MANUAL | From Phase 3 |
| CVCS | MANUAL | THIS PHASE |
| RCPs (4) | MANUAL | THIS PHASE |
| Steam Generators | AUTO | Holds 1000 psia |
| Turbine | AUTO | Follows load |

# 2. CVCS Model

## 2.1 Key Feature: Boron Transport Delay

Critical: Changes in VCT boron take 5-15 minutes to reach core. Model as pipe delay + mixing.

const float CVCS_TRANSPORT_TIME = 600f; // seconds

// Use ring buffer to track boron in transit

## 2.2 Flow Rates

| Parameter | Value |
| --- | --- |
| Normal Letdown | 75 gpm |
| Normal Charging | 87 gpm (55 to RCS + 32 to seals) |
| VCT Volume | ~1000 gallons |
| Boric Acid Stock | 7000 ppm |

# 3. RCP Model

## 3.1 Pump Parameters

| Parameter | Value |
| --- | --- |
| Pumps | 4 (one per loop) |
| Flow per pump | 97,600 gpm |
| Nominal speed | 1189 rpm |
| Coastdown τ | ~12 seconds |

## 3.2 Coastdown Model

void UpdateRCP(ref RCPState pump, float dt) {

if (!pump.running) {

pump.speed *= Mathf.Exp(-dt / 12f);

pump.flow = 97600f * pump.speed / 1189f;

}

}

# 4. Phase 4 Test Suite

## 4.1 Unit Tests

| ID | Test Case | Pass |
| --- | --- | --- |
| CV-001 | Boron transport delay measurable (5-15 min) | □ |
| CV-002 | VCT level responds to charging/letdown balance | □ |
| CV-003 | 100 ppm dilution = ~900 pcm reactivity | □ |
| CV-004 | Charging flow to RCP seals = 8 gpm/pump | □ |
| RCP-001 | Coastdown τ = 12 ± 3 seconds | □ |
| RCP-002 | 1 pump trip → ~75% flow | □ |
| RCP-003 | Affinity laws: Q ∝ N, H ∝ N² | □ |
| RCP-004 | Flow trip at <87% triggers reactor trip | □ |

## 4.2 Integration Tests

| ID | Test Case | Pass |
| --- | --- | --- |
| INT-301 | Dilution at 30 gpm shows delayed reactivity rise | □ |
| INT-302 | Single RCP trip: flow drops, ΔT increases | □ |
| INT-303 | Loss of charging: PZR level drops | □ |
| INT-304 | Boron from VCT reaches core in ~10 min | □ |
| INT-305 | 4 RCPs running = 390,400 gpm total | □ |
| INT-306 | RCP trip at power triggers low flow trip | □ |

# 5. Phase 4 Exit Gate

| Requirement | Status | Date |
| --- | --- | --- |
| Boron transport delay of 5-15 minutes verified | □ PASS | ______ |
| RCP coastdown τ = 12 ± 3 seconds | □ PASS | ______ |
| Single RCP trip reduces flow to ~75% | □ PASS | ______ |
| Low flow trip functions correctly | □ PASS | ______ |
| All 8 unit tests pass | □ PASS | ______ |
| All 6 integration tests pass | □ PASS | ______ |
| PHASE 4 GATE APPROVED | □ YES | ______ |

Phase 4 Gate Passed → Proceed to Phase 5: Steam Generators