CRITICAL

MASTER THE ATOM

PHASE 5: STEAM GENERATORS

Secondary Side Heat Removal

Unity 6.3 Implementation | Duration: 3-4 weeks

Gate: Heat Transfer & Shrink/Swell Validated

# 1. Phase Overview

Phase 5 closes the thermal loop: reactor heat must now be actively removed by SGs or RCS temp will rise.

## 1.1 Entry Criteria

✓ Phase 4 Gate PASSED

✓ CVCS boron delay and RCP coastdown working

## 1.2 System Status After Phase 5

| System | Mode | Notes |
| --- | --- | --- |
| Reactor Core | MANUAL | From Phase 2 |
| Pressurizer | MANUAL | From Phase 3 |
| CVCS | MANUAL | From Phase 4 |
| RCPs (4) | MANUAL | From Phase 4 |
| Steam Generators (4) | MANUAL | THIS PHASE |
| Turbine | AUTO | Follows load |

# 2. Steam Generator Physics

## 2.1 Key Parameters (Model F)

| Parameter | Value |
| --- | --- |
| Number of SGs | 4 (one per loop) |
| Heat transfer area | ~55,000 ft² per SG |
| Full power steam flow | 3.8 × 10⁶ lb/hr per SG |
| Steam pressure (100%) | 1000 psia |
| Feedwater temperature | 440°F |

## 2.2 Heat Transfer

// Q = U × A × LMTD

float LMTD = (Thot - Tsteam) - (Tcold - Tsteam);

float Q = UA * LMTD;

## 2.3 Shrink and Swell

Critical behavior: Level initially moves OPPOSITE to expectation due to void changes.

SHRINK: Power ↑ → more boiling → voids ↑ → level DROPS (even though inventory unchanged)

SWELL: Power ↓ → less boiling → voids ↓ → level RISES (even though inventory unchanged)

# 3. Phase 5 Test Suite

## 3.1 Unit Tests

| ID | Test Case | Pass |
| --- | --- | --- |
| SG-001 | Heat transfer matches power at steady state | □ |
| SG-002 | Steam pressure rises if steam flow < generation | □ |
| SG-003 | Feedwater controller maintains 50% level | □ |
| SG-004 | Shrink on power increase (level drops 5-10%) | □ |
| SG-005 | Swell on power decrease (level rises 5-10%) | □ |
| SG-006 | Steam dump modulates in Tavg mode | □ |
| SG-007 | Primary outlet temp calculated correctly | □ |
| SG-008 | LO-LO level triggers reactor trip | □ |

## 3.2 Integration Tests

| ID | Test Case | Pass |
| --- | --- | --- |
| INT-401 | 10% load increase: pressure dip ~30 psi | □ |
| INT-402 | Turbine trip: pressure rises to ~1050 psia | □ |
| INT-403 | No safety valve lift (<1100 psia) | □ |
| INT-404 | Shrink recovers in 2-3 minutes | □ |
| INT-405 | RCS temp rises if SG heat removal insufficient | □ |
| INT-406 | Cooldown using steam dump at 50°F/hr | □ |

# 4. Phase 5 Exit Gate

| Requirement | Status | Date |
| --- | --- | --- |
| Heat transfer matches power at steady state | □ PASS | ______ |
| Shrink/swell behavior validated | □ PASS | ______ |
| Steam pressure stable at 1000±20 psia | □ PASS | ______ |
| Feedwater maintains level at 50±5% | □ PASS | ______ |
| No safety valve lift on turbine trip | □ PASS | ______ |
| All 8 unit tests pass | □ PASS | ______ |
| All 6 integration tests pass | □ PASS | ______ |
| PHASE 5 GATE APPROVED | □ YES | ______ |

Phase 5 Gate Passed → Proceed to Phase 6: Turbine/Generator