CRITICAL

MASTER THE ATOM

PHASE 6: TURBINE/GENERATOR

Complete System Integration

Unity 6.3 Implementation | Duration: 2-3 weeks

Gate: Full Plant Operation Validated

# 1. Phase Overview

Phase 6 completes Critical. ALL systems are now MANUAL - operator controls the entire power chain.

## 1.1 Entry Criteria

✓ Phase 5 Gate PASSED

✓ Steam generators respond to load changes

## 1.2 Final System Status

| System | Mode | Notes |
| --- | --- | --- |
| Reactor Core | MANUAL | From Phase 2 |
| Pressurizer | MANUAL | From Phase 3 |
| CVCS | MANUAL | From Phase 4 |
| RCPs (4) | MANUAL | From Phase 4 |
| Steam Generators (4) | MANUAL | From Phase 5 |
| Turbine/Generator | MANUAL | THIS PHASE |

# 2. Turbine Model

## 2.1 Key Parameters

| Parameter | Value |
| --- | --- |
| Rated output | ~1150 MW |
| Rated speed | 1800 rpm (60 Hz) |
| Steam flow (rated) | 15.2 × 10⁶ lb/hr |
| Inlet pressure | 1000 psia |
| Efficiency | ~34% |

## 2.2 Governor Control

// Before sync: speed control

// After sync: load control

if (breaker == OPEN) {

// Control speed to 1800 rpm

} else {

// Control load to setpoint

}

## 2.3 Grid Sync Requirements

Frequency: Within 0.05 Hz of 60 Hz

Voltage: Within 5% of grid

Phase angle: Within 10 degrees

# 3. Complete Plant Integration

## 3.1 Energy Flow

Reactor (fission) → Coolant → SGs → Steam → Turbine → Generator → Grid

## 3.2 Load Following Dynamics

1. Operator increases load setpoint

2. Governor opens valves → more steam demand

3. Steam pressure drops → SG heat transfer increases

4. Tavg drops → positive MTC → power rises

5. New equilibrium at higher load

# 4. Phase 6 Test Suite

## 4.1 Unit Tests

| ID | Test Case | Pass |
| --- | --- | --- |
| TB-001 | Governor maintains 1800 rpm before sync | □ |
| TB-002 | Load control works after sync | □ |
| TB-003 | Droop characteristic: 5% speed drop = 100% load | □ |
| TB-004 | Generator output = turbine power × 0.985 | □ |
| TB-005 | Frequency = speed / 30 | □ |
| TB-006 | Sync check: freq, voltage, phase angle | □ |
| TB-007 | Load ramp rate limited to 5%/min | □ |
| TB-008 | Turbine trip closes all valves | □ |

## 4.2 Integration Tests

| ID | Test Case | Pass |
| --- | --- | --- |
| INT-501 | Complete startup: cold → 100% power | □ |
| INT-502 | Load follow: 100% → 50% → 100% with xenon | □ |
| INT-503 | Generator trip (load rejection): no damage | □ |
| INT-504 | Turbine trip: reactor trips, plant stabilizes | □ |
| INT-505 | Grid sync successful | □ |
| INT-506 | Steady state: all parameters within tolerance | □ |

# 5. Final Acceptance Criteria

| Parameter | Value @ 100% | Tolerance |
| --- | --- | --- |
| Reactor Power | 3411 MW | ±20 MW |
| Generator Output | ~1150 MW | ±20 MW |
| Tavg | 588°F | ±2°F |
| RCS Pressure | 2235 psig | ±10 psig |
| Steam Pressure | 1000 psia | ±20 psia |
| Frequency | 60.00 Hz | ±0.02 Hz |

# 6. Phase 6 Exit Gate (PROJECT COMPLETE)

| Requirement | Status | Date |
| --- | --- | --- |
| Complete startup: cold shutdown → 100% | □ PASS | ______ |
| Load follow with xenon management | □ PASS | ______ |
| Reactor trip from 100% - plant stabilizes | □ PASS | ______ |
| Turbine trip - no equipment damage | □ PASS | ______ |
| All steady-state parameters within tolerance | □ PASS | ______ |
| All 8 unit tests pass | □ PASS | ______ |
| All 6 integration tests pass | □ PASS | ______ |
| PHASE 6 GATE APPROVED | □ YES | ______ |

# 7. CRITICAL: Master the Atom - COMPLETE

✓ Validated physics engine (Phase 1)

✓ Reactor core with point kinetics and feedback (Phase 2)

✓ Pressurizer with steam dynamics (Phase 3)

✓ CVCS with boron transport delay (Phase 4)

✓ RCPs with coastdown dynamics (Phase 4)

✓ Steam generators with shrink/swell (Phase 5)

✓ Turbine-generator with governor control (Phase 6)

✓ Complete protection system

The simulator provides a realistic, challenging experience for understanding PWR operations.