CRITICAL

MASTER THE ATOM

PHASE 3: PRESSURIZER SYSTEM

Complete Implementation with Phase 1 Physics Integration

Duration: 3-4 weeks | Unit Tests: 78 | Gate: All Tests Pass

# 1. PHASE OVERVIEW

Phase 3 replaces the AUTO pressurizer with a physics-based model using Phase 1 physics library.

This phase implements 9 of 13 Gap Analysis fixes (Gaps #1-4, #6-10). The pressurizer is the first system that 'fights back' with real dynamics.

## 1.1 Entry Criteria

✓ Phase 1 Gate PASSED (all 112 physics tests)

✓ Phase 2 Gate PASSED (reactor achieves criticality)

## 1.2 System Status

| System | Mode | Notes |
| --- | --- | --- |
| Reactor Core | MANUAL | From Phase 2 |
| Pressurizer | MANUAL | THIS PHASE |
| Steam Generators | AUTO | Holds 1000 psia |
| CVCS/RCPs/Turbine | AUTO | Perfect boundary conditions |

# 2. PHASE 1 PHYSICS DEPENDENCIES

Phase 3 is the primary consumer of Phase 1 physics. Every calculation flows through validated modules.

| Phase 1 Module | Phase 3 Usage | Gap # |
| --- | --- | --- |
| PlantConstants.cs | All PZR reference values | #5 |
| WaterProperties.cs | Saturation, enthalpy, density | #11,12 |
| SteamThermodynamics.cs | Void fraction, steam quality | #13 |
| ThermalMass.cs | 200,000 lb wall thermal mass | - |
| ThermalExpansion.cs | RCS surge volume calculation | - |
| CoupledThermo.cs | ALL pressure calculations | #1 |
| PressurizerPhysics.cs | Flash, spray, wall, rainout, heater | #2-4,6-8 |
| FluidFlow.cs | Surge line hydraulics | #9 |
| HeatTransfer.cs | Enthalpy transport for surge | #10 |

## 2.1 Critical Module Call Sequence

1. ThermalExpansion.SurgeVolume(rcsVolume, ΔTavg, P)

2. FluidFlow.SurgeLineFlow(ΔP, diameter, length) [GAP #9]

3. HeatTransfer.EnthalpyTransport(massFlow, h_surge, h_sat) [GAP #10]

4. PressurizerPhysics.ThreeRegionModel(state) [GAP #4]

5. CoupledThermo.SolveEquilibrium(state, ΔT) [GAP #1] ← ITERATIVE

6. PressurizerPhysics.FlashEvaporation(...) [GAP #2]

7. PressurizerPhysics.HeaterSteamGeneration(...) [GAP #8] τ=20s

8. PressurizerPhysics.SprayCondensation(...) [GAP #3] η=85%

9. PressurizerPhysics.WallCondensation(...) [GAP #6]

10. PressurizerPhysics.Rainout(...) [GAP #7]

CRITICAL: Never bypass CoupledThermo.SolveEquilibrium() - simple PV^n is WRONG for closed systems.

# 3. GAP ANALYSIS IMPLEMENTATION

Phase 3 implements 9 of 13 physics gaps identified in Phase 1 review.

| Gap | Issue | Without Fix | With Fix |
| --- | --- | --- | --- |
| #1 | P-T-V coupling | 10°F rise → 0 psi (wrong) | 10°F rise → 60-80 psi |
| #2 | Flash evaporation | Rapid pressure drop | Self-regulating retard |
| #3 | Spray condensation | Instant, 100% efficient | Time-based, 85% η |
| #4 | Three-region model | Simple 2-phase | Subcooled mixing |
| #6 | Wall condensation | No wall effects | 200,000 lb thermal mass |
| #7 | Rainout | No bulk condensation | Vapor subcool → rain |
| #8 | Heater lag | Instant response | τ=20s exponential |
| #9 | Surge line | Instant, no resistance | 14" pipe, friction |
| #10 | Enthalpy transport | No energy tracking | 60 BTU/lb deficit |

# 4. PRESSURIZER REFERENCE PARAMETERS

All values from PlantConstants.cs (Phase 1, Westinghouse 4-loop FSAR)

## 4.1 Physical Dimensions

| Parameter | Value | Constant |
| --- | --- | --- |
| Total Volume | 1800 ft³ | PZR_VOLUME_TOTAL |
| Water Volume (60%) | 1080 ft³ | PZR_VOLUME_WATER |
| Steam Volume (40%) | 720 ft³ | PZR_VOLUME_STEAM |
| Wall Mass (steel) | 200,000 lb | PZR_WALL_MASS |
| Height | 52.75 ft | PZR_HEIGHT |

## 4.2 Heater System

| Parameter | Value | Constant |
| --- | --- | --- |
| Total Power | 1800 kW | HEATER_POWER_TOTAL |
| Proportional | 500 kW | HEATER_POWER_PROP |
| Backup | 1300 kW | HEATER_POWER_BACKUP |
| Time Constant (τ) | 20 seconds | HEATER_TAU |

## 4.3 Spray System

| Parameter | Value | Constant |
| --- | --- | --- |
| Max Flow | 900 gpm | SPRAY_FLOW_MAX |
| Temperature | 558°F (=Tcold) | SPRAY_TEMP |
| Efficiency | 85% | SPRAY_EFFICIENCY |

## 4.4 Control Setpoints

| Setpoint | Pressure | Action |
| --- | --- | --- |
| Spray Full ON | >2280 psig | 900 gpm spray |
| Spray ON | >2260 psig | Proportional spray |
| Normal | 2235 psig | Proportional heaters |
| Heaters Full ON | <2210 psig | All 1800 kW |
| PORV Open | >2335 psig | Relief valve lifts |
| Safety Valve | >2485 psig | Code safety opens |

# 5. UNIT TEST SUITE (54 Tests)

## 5.1 Phase 1 Integration Tests (24 tests)

Verify Phase 3 correctly uses Phase 1 physics modules

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| P1-01 | CoupledThermo.SolveEquilibrium returns valid pressure | 2200-2400 psig | □ |
| P1-02 | 10°F Tavg rise → 60-80 psi increase (NOT 0) | 60-80 psi | □ |
| P1-03 | Iterative solver converges <10 iterations | <10 iter | □ |
| P1-04 | FlashEvaporation positive on depressurization | >0 lb/s | □ |
| P1-05 | SprayCondensation at 900 gpm → ~5 lb/s | 4-6 lb/s | □ |
| P1-06 | Spray efficiency = 85% (not 100%) | η=0.85 | □ |
| P1-07 | WallCondensation continuous at steady state | >0 | □ |
| P1-08 | Rainout when steamTemp < satTemp | >0 lb/s | □ |
| P1-09 | HeaterSteamGeneration with τ=20s lag | 63% @ 20s | □ |
| P1-10 | ThreeRegionModel tracks subcooled mass | >0 lb | □ |
| P1-11 | SurgeLineFlow proportional to √ΔP | √ΔP | □ |
| P1-12 | EnthalpyTransport negative for insurge | <0 BTU/s | □ |

... (12 more Phase 1 integration tests covering all module interfaces)

## 5.2 Pressurizer State Tests (12 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| PZ-01 | Level = liquidVolume / 1800 × 100 | Formula | □ |
| PZ-02 | Mass conservation: total mass constant | Δm ≈ 0 | □ |
| PZ-03 | Energy conservation validated | ΔE ≈ 0 | □ |
| PZ-04 | Surge IN → level increases | dL > 0 | □ |
| PZ-05 | Surge OUT → level decreases | dL < 0 | □ |
| PZ-06 | Heater ON → pressure rises | dP > 0 | □ |
| PZ-07 | Spray ON → pressure drops | dP < 0 | □ |
| PZ-08 | Steam bubble maintained (volume > 0) | V_s > 0 | □ |

... (4 more state tests)

## 5.3 Control System Tests (10 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| CT-01 | Heaters OFF when P > 2235 psig | 0 kW | □ |
| CT-02 | Full heaters at P < 2210 psig | 1800 kW | □ |
| CT-03 | Spray OFF when P < 2260 psig | 0 gpm | □ |
| CT-04 | Full spray at P > 2280 psig | 900 gpm | □ |
| CT-05 | PORV opens at 2335 psig | Open | □ |
| CT-06 | PORV closes at 2300 psig (hysteresis) | Closed | □ |
| CT-07 | Safety valve opens at 2485 psig | Open | □ |

... (3 more control tests)

## 5.4 Relief Valve Tests (8 tests)

PORV and Safety valve logic validation

# 6. INTEGRATION TEST SUITE (24 Tests)

## 6.1 Transient Response Tests (12 tests)

| ID | Scenario | Success Criteria | □ |
| --- | --- | --- | --- |
| INT-301 | 10% power increase (50→60%) | Pressure spike ~20 psi, recovers <60s | □ |
| INT-302 | 10% power decrease (100→90%) | Pressure drop ~20 psi, recovers <60s | □ |
| INT-303 | 25% power increase (50→75%) | Pressure spike ~50 psi, spray activates | □ |
| INT-304 | Reactor trip from 100% | Pressure spike then drops, heaters recover | □ |
| INT-305 | 100% load rejection | No PORV lift (<2335 psig) | □ |
| INT-306 | Loss of all heaters | Slow pressure decay, operator must respond | □ |
| INT-307 | Stuck open PORV | Rapid depressurization, flash evap visible | □ |
| INT-308 | Level tracks programmed value (25-60%) | Level follows Tavg program | □ |

... (4 more transient tests)

## 6.2 Steady State Tests (6 tests)

| ID | Condition | Tolerance | □ |
| --- | --- | --- | --- |
| SS-01 | Pressure at 100% power | 2235 ± 5 psig | □ |
| SS-02 | Level at 100% power | 60 ± 3% | □ |
| SS-03 | Heater power at equilibrium | ~300 kW (losses) | □ |
| SS-04 | Pressure drift rate | < 5 psi/hour | □ |
| SS-05 | Steam bubble maintained | Volume > 0 always | □ |
| SS-06 | Wall temperature stable | < 1°F/hour drift | □ |

## 6.3 Coupling Validation Tests (6 tests)

Verify Phase 1 physics coupling is correctly implemented

# 7. PHASE 3 EXIT GATE

| Requirement | Status | Date |
| --- | --- | --- |
| All 24 Phase 1 integration tests pass | □ PASS | ________ |
| All 12 pressurizer state tests pass | □ PASS | ________ |
| All 10 control system tests pass | □ PASS | ________ |
| All 8 relief valve tests pass | □ PASS | ________ |
| All 24 integration tests pass | □ PASS | ________ |
| Pressure responds to thermal transients (±20 psi for 10% step) | □ PASS | ________ |
| Heater lag τ=20s verified | □ PASS | ________ |
| Flash evaporation retards pressure drop | □ PASS | ________ |
| Spray efficiency 85% verified | □ PASS | ________ |
| Steam bubble maintained (no solid PZR) | □ PASS | ________ |
| PHASE 3 GATE APPROVED | □ YES | ________ |

Phase 3 Gate Passed → Proceed to Phase 4: CVCS & RCPs