CRITICAL

MASTER THE ATOM

PHASE 4: CVCS & RCPs

Boron Transport & Pump Dynamics with Phase 1 Physics

Duration: 2-3 weeks | Unit Tests: 65 | Gate: All Tests Pass

# 1. PHASE OVERVIEW

Phase 4 adds CVCS (boron control with transport delay) and RCPs (flow dynamics with coastdown).

Key Phase 1 modules used: FluidFlow.cs (RCP coastdown, natural circulation), ReactorKinetics.cs (boron worth), WaterProperties.cs (density for boron transport).

## 1.1 Entry Criteria

✓ Phase 3 Gate PASSED (pressurizer responds to thermal transients)

## 1.2 System Status

| System | Mode | Notes |
| --- | --- | --- |
| Reactor Core | MANUAL | From Phase 2 |
| Pressurizer | MANUAL | From Phase 3 |
| CVCS | MANUAL | THIS PHASE - Boron delay |
| RCPs (4) | MANUAL | THIS PHASE - Coastdown |
| Steam Generators | AUTO | Holds 1000 psia |
| Turbine | AUTO | Follows load |

# 2. PHASE 1 PHYSICS DEPENDENCIES

| Phase 1 Module | Phase 4 Usage |
| --- | --- |
| PlantConstants.cs | RCP specs: 97,600 gpm/pump, τ=12s coastdown, CVCS flows |
| FluidFlow.cs | PumpCoastdown(), AffinityLaws(), NaturalCirculation() |
| ReactorKinetics.cs | BoronReactivity() → -9 pcm/ppm worth calculation |
| WaterProperties.cs | LiquidDensity() for boron concentration tracking |
| HeatTransfer.cs | RCP heat addition: 21 MW per running pump |

## 2.1 Critical: Boron Transport Delay

Changes in VCT boron take 5-15 minutes to reach the core.

const float CVCS_TRANSPORT_TIME = 600f;  // ~10 minutes

// Use ring buffer to track boron in transit

state.boronPipeline = new Queue<BoronPacket>();

Without this delay: Dilution causes INSTANT reactivity → unrealistic power excursion

With this delay: Dilution takes 10 min to affect core → operator has time to respond

## 2.2 Critical: RCP Coastdown

RCP coastdown uses FluidFlow.PumpCoastdown() with τ=12 seconds.

void UpdateRCP(ref RCPState pump, float dt) {

if (!pump.running) {

pump.speed = FluidFlow.PumpCoastdown(pump.speed, dt, TAU_RCP);

pump.flow = FluidFlow.AffinityLaws_Flow(pump.speed, NOMINAL_SPEED, NOMINAL_FLOW);

}

}

# 3. REFERENCE PARAMETERS

## 3.1 CVCS Parameters

| Parameter | Value | Constant |
| --- | --- | --- |
| Normal Letdown | 75 gpm | LETDOWN_NORMAL |
| Normal Charging | 87 gpm total | CHARGING_NORMAL |
| Charging to RCS | 55 gpm | CHARGING_TO_RCS |
| Seal Injection | 32 gpm (8/pump) | SEAL_INJECTION |
| VCT Volume | ~1000 gallons | VCT_VOLUME |
| Boric Acid Stock | 7000 ppm | BORIC_ACID_CONC |
| Transport Time | ~10 minutes | TRANSPORT_TIME |

## 3.2 RCP Parameters

| Parameter | Value | Constant |
| --- | --- | --- |
| Number of Pumps | 4 (one per loop) | RCP_COUNT |
| Flow per Pump | 97,600 gpm | RCP_FLOW |
| Total RCS Flow | 390,400 gpm | RCS_FLOW_TOTAL |
| Nominal Speed | 1189 rpm | RCP_SPEED |
| Coastdown τ | 12 seconds | RCP_COASTDOWN_TAU |
| Heat Addition | 21 MW per pump | RCP_HEAT |
| Low Flow Trip | <87% flow | LOW_FLOW_TRIP |

# 4. UNIT TEST SUITE (41 Tests)

## 4.1 CVCS Tests (20 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| CV-01 | Boron transport delay measurable (5-15 min) | 5-15 min | □ |
| CV-02 | VCT level responds to charging/letdown | Level Δ | □ |
| CV-03 | 100 ppm dilution = ~900 pcm reactivity | ~900 pcm | □ |
| CV-04 | ReactorKinetics.BoronReactivity(-9 pcm/ppm) | -9 pcm/ppm | □ |
| CV-05 | Boron from VCT reaches core in ~10 min | ~10 min | □ |
| CV-06 | Ring buffer tracks boron packets correctly | FIFO | □ |

... (14 more CVCS tests)

## 4.2 RCP Tests (21 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| RCP-01 | Coastdown τ = 12 ± 3 seconds | 9-15s | □ |
| RCP-02 | 1 pump trip → ~75% flow | ~75% | □ |
| RCP-03 | FluidFlow.AffinityLaws: Q ∝ N, H ∝ N² | Affinity | □ |
| RCP-04 | Flow trip at <87% triggers reactor trip | Trip | □ |
| RCP-05 | 4 RCPs running = 390,400 gpm total | 390,400 | □ |
| RCP-06 | Natural circulation transition modeled | >0 gpm | □ |

... (15 more RCP tests)

# 5. INTEGRATION TEST SUITE (24 Tests)

| ID | Scenario | Success Criteria | □ |
| --- | --- | --- | --- |
| INT-401 | Dilution at 30 gpm shows delayed reactivity | Delay visible | □ |
| INT-402 | Single RCP trip: flow drops, ΔT increases | ΔT up, flow down | □ |
| INT-403 | Loss of charging: PZR level drops | Level drops | □ |
| INT-404 | RCP trip at power triggers low flow trip | Rx trip | □ |
| INT-405 | Natural circulation established after all RCP loss | Flow > 0 | □ |
| INT-406 | Boration from 1000→1100 ppm takes ~15 min | ~15 min | □ |

... (18 more integration tests)

# 6. PHASE 4 EXIT GATE

| Requirement | Status | Date |
| --- | --- | --- |
| Boron transport delay 5-15 minutes verified | □ PASS | ________ |
| RCP coastdown τ = 12 ± 3 seconds | □ PASS | ________ |
| Single RCP trip reduces flow to ~75% | □ PASS | ________ |
| Low flow trip functions correctly | □ PASS | ________ |
| Natural circulation transition works | □ PASS | ________ |
| All 41 unit tests pass | □ PASS | ________ |
| All 24 integration tests pass | □ PASS | ________ |
| PHASE 4 GATE APPROVED | □ YES | ________ |

Phase 4 Gate Passed → Proceed to Phase 5: Steam Generators