CRITICAL

MASTER THE ATOM

PHASE 5: STEAM GENERATORS

Heat Transfer & Shrink/Swell with Phase 1 Physics

Duration: 3-4 weeks | Unit Tests: 72 | Gate: All Tests Pass

# 1. PHASE OVERVIEW

Phase 5 closes the thermal loop: reactor heat must now be actively removed by SGs.

Key Phase 1 modules: HeatTransfer.cs (LMTD, UA), WaterProperties.cs (secondary saturation), ThermalMass.cs (tube bundle), CoupledThermo.cs (RCS response).

## 1.1 Critical Coupling

SGs couple primary and secondary sides:

• SGs remove LESS heat than reactor produces → RCS heats up → pressure rises

• SGs remove MORE heat than reactor produces → RCS cools → pressure drops

This creates the fundamental load-following challenge.

# 2. PHASE 1 PHYSICS DEPENDENCIES

| Phase 1 Module | Phase 5 Usage |
| --- | --- |
| PlantConstants.cs | SG parameters: 55,000 ft² area, 4 SGs, Model F specs |
| WaterProperties.cs | SaturationTemperature(1000 psia) = 545°F secondary |
| HeatTransfer.cs | LMTD(), UACalculation() for primary-secondary transfer |
| ThermalMass.cs | Tube bundle thermal mass for lag effects |
| CoupledThermo.cs | RCS temperature/pressure response to heat removal |

## 2.1 Heat Transfer Model

// Q = U × A × LMTD

float LMTD = HeatTransfer.LMTD(Thot, Tcold, Tsteam, Tsteam);

float Q = UA * LMTD;  // UA sized for design conditions

## 2.2 Shrink and Swell

CRITICAL: Level initially moves OPPOSITE to expectation due to void changes.

SHRINK: Power↑ → more boiling → voids↑ → level DROPS (inventory unchanged)

SWELL: Power↓ → less boiling → voids↓ → level RISES (inventory unchanged)

# 3. STEAM GENERATOR PARAMETERS (Model F)

| Parameter | Value | Constant |
| --- | --- | --- |
| Number of SGs | 4 (one per loop) | SG_COUNT |
| Heat Transfer Area | ~55,000 ft² each | SG_AREA |
| U-tube Count | ~5,600 tubes | SG_TUBES |
| Full Power Steam Flow | 3.8×10⁶ lb/hr each | SG_STEAM_FLOW |
| Steam Pressure (100%) | 1000 psia | SG_STEAM_PRESS |
| Steam Temperature | 545°F (saturation) | SG_STEAM_TEMP |
| Feedwater Temperature | 440°F | SG_FW_TEMP |

# 4. UNIT TEST SUITE (48 Tests)

## 4.1 Heat Transfer Tests (16 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| HT-01 | Heat transfer matches power at steady state | 3411 MW total | □ |
| HT-02 | HeatTransfer.LMTD calculated correctly | ~40°F | □ |
| HT-03 | Primary outlet temp (Tcold) from heat balance | 558°F | □ |
| HT-04 | Steam pressure rises if flow < generation | dP > 0 | □ |

... (12 more heat transfer tests)

## 4.2 Shrink/Swell Tests (12 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| SS-01 | Shrink on power increase (level drops 5-10%) | Level drops | □ |
| SS-02 | Swell on power decrease (level rises 5-10%) | Level rises | □ |
| SS-03 | Shrink recovers in 2-3 minutes | Recovery | □ |
| SS-04 | Void fraction proportional to power | Linear | □ |

... (8 more shrink/swell tests)

## 4.3 Level/Pressure Control Tests (20 tests)

# 5. INTEGRATION TEST SUITE (24 Tests)

| ID | Scenario | Success Criteria | □ |
| --- | --- | --- | --- |
| INT-501 | 10% load increase: pressure dip ~30 psi | Recovery <60s | □ |
| INT-502 | Turbine trip: pressure rises to ~1050 psia | No safety lift | □ |
| INT-503 | Shrink on power increase recovers | 2-3 min | □ |
| INT-504 | RCS temp rises if SG heat removal insufficient | Tavg rises | □ |
| INT-505 | Cooldown using steam dump at 50°F/hr | Controlled | □ |
| INT-506 | LO-LO level triggers reactor trip | Rx trip | □ |

... (18 more integration tests)

# 6. PHASE 5 EXIT GATE

| Requirement | Status | Date |
| --- | --- | --- |
| Heat transfer matches power at steady state | □ PASS | ________ |
| Shrink/swell behavior validated | □ PASS | ________ |
| Steam pressure stable at 1000±20 psia | □ PASS | ________ |
| Feedwater maintains level at 50±5% | □ PASS | ________ |
| No safety valve lift on turbine trip | □ PASS | ________ |
| All 48 unit tests pass | □ PASS | ________ |
| All 24 integration tests pass | □ PASS | ________ |
| PHASE 5 GATE APPROVED | □ YES | ________ |

Phase 5 Gate Passed → Proceed to Phase 6: Turbine/Generator