CRITICAL

MASTER THE ATOM

PHASE 6: TURBINE/GENERATOR

Complete Plant Integration - ALL Systems MANUAL

Duration: 2-3 weeks | Unit Tests: 68 | Gate: Full Plant Validated

# 1. PHASE OVERVIEW

Phase 6 completes Critical. ALL systems are now MANUAL - operator controls the entire power chain.

## 1.1 Final System Status

| System | Mode | Phase |
| --- | --- | --- |
| Reactor Core | MANUAL | Phase 2 |
| Pressurizer | MANUAL | Phase 3 |
| CVCS | MANUAL | Phase 4 |
| RCPs (4) | MANUAL | Phase 4 |
| Steam Generators (4) | MANUAL | Phase 5 |
| Turbine/Generator | MANUAL | THIS PHASE |

## 1.2 Complete Energy Flow

Reactor (fission) → Coolant → SGs → Steam → Turbine → Generator → Grid

Each link must be balanced. Imbalances cause temperature, pressure, or frequency deviations.

# 2. TURBINE/GENERATOR PARAMETERS

| Parameter | Value |
| --- | --- |
| Rated Output | ~1150 MW electrical |
| Rated Speed | 1800 rpm (60 Hz, 4-pole) |
| Steam Flow (rated) | 15.2×10⁶ lb/hr |
| Inlet Pressure | 1000 psia |
| Efficiency | ~34% |
| Droop | 5% (5% speed drop = 100% load) |

## 2.1 Grid Sync Requirements

• Frequency: Within 0.05 Hz of 60 Hz

• Voltage: Within 5% of grid

• Phase angle: Within 10 degrees

## 2.2 Load Following Dynamics

1. Operator increases load setpoint

2. Governor opens valves → more steam demand

3. Steam pressure drops → SG heat transfer increases

4. Tavg drops → positive MTC → power rises

5. New equilibrium at higher load

# 3. UNIT TEST SUITE (44 Tests)

## 3.1 Turbine Tests (20 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| TB-01 | Governor maintains 1800 rpm before sync | 1800 rpm | □ |
| TB-02 | Load control works after sync | MW matches | □ |
| TB-03 | Droop: 5% speed drop = 100% load | 5% droop | □ |
| TB-04 | Generator output = turbine × 0.985 | η = 98.5% | □ |
| TB-05 | Frequency = speed / 30 (4-pole) | 60 Hz | □ |
| TB-06 | Sync check validates freq, volt, phase | All 3 | □ |

... (14 more turbine tests)

## 3.2 Generator & Grid Tests (24 tests)

# 4. INTEGRATION TEST SUITE (24 Tests)

## 4.1 Full Plant Tests

| ID | Scenario | Success Criteria | □ |
| --- | --- | --- | --- |
| INT-601 | Complete startup: cold → 100% power | No trips | □ |
| INT-602 | Load follow: 100% → 50% → 100% with xenon | Manages xenon | □ |
| INT-603 | Generator trip (load rejection): no damage | Plant stable | □ |
| INT-604 | Turbine trip: reactor trips, plant stabilizes | Safe state | □ |
| INT-605 | Grid sync successful | Sync achieved | □ |
| INT-606 | Steady state: all parameters within tolerance | All pass | □ |

... (18 more integration tests)

## 4.2 Final Acceptance Criteria

| Parameter | Value @ 100% | Tolerance |
| --- | --- | --- |
| Reactor Power | 3411 MW | ±20 MW |
| Generator Output | ~1150 MW | ±20 MW |
| Tavg | 588°F | ±2°F |
| RCS Pressure | 2235 psig | ±10 psig |
| Steam Pressure | 1000 psia | ±20 psia |
| Frequency | 60.00 Hz | ±0.02 Hz |

# 5. PHASE 6 EXIT GATE (PROJECT COMPLETE)

| Requirement | Status | Date |
| --- | --- | --- |
| Complete startup: cold shutdown → 100% | □ PASS | ________ |
| Load follow with xenon management | □ PASS | ________ |
| Reactor trip from 100% - plant stabilizes | □ PASS | ________ |
| Turbine trip - no equipment damage | □ PASS | ________ |
| All steady-state parameters within tolerance | □ PASS | ________ |
| All 44 unit tests pass | □ PASS | ________ |
| All 24 integration tests pass | □ PASS | ________ |
| PHASE 6 GATE APPROVED | □ YES | ________ |

# CRITICAL: MASTER THE ATOM - COMPLETE

✓ Validated physics engine with 13 gap fixes (Phase 1)

✓ Reactor core with point kinetics and feedback (Phase 2)

✓ Pressurizer with non-equilibrium dynamics (Phase 3)

✓ CVCS with boron transport delay (Phase 4)

✓ RCPs with coastdown dynamics (Phase 4)

✓ Steam generators with shrink/swell (Phase 5)

✓ Turbine-generator with governor control (Phase 6)

✓ Complete protection system

The simulator provides a realistic, challenging experience for understanding PWR operations.