CRITICAL

MASTER THE ATOM

PHASE 1

CORE PHYSICS ENGINE

Complete Implementation Specification

Westinghouse 4-Loop PWR Reference (3411 MWt)

Duration: 3-4 weeks  |  Unit Tests: 112  |  Gate: 100% Pass

# 1. PHASE OVERVIEW

## 1.1 Purpose

Phase 1 establishes the foundational physics library that ALL Critical systems depend on. Pure C# library with comprehensive unit tests - no Unity GameObjects, no MonoBehaviours.

An error in Phase 1 cascades into every subsequent phase. By isolating physics into a tested library first, we guarantee correct foundations.

## 1.2 Critical Physics Principles

The PWR thermal-hydraulic system is COUPLED:

• Temperature change → Volume change → Pressure change → Property change → Temperature response

• Pressurizer steam bubble involves flash evaporation, spray condensation, wall effects, non-equilibrium regions

• Enthalpy transport matters - surge water (619°F) vs saturated water (653°F at 2250 psia)

• Simple polytropic compression is INSUFFICIENT for realistic pressurizer response

## 1.3 Physics Gaps Addressed (13 Total)

| Gap | Issue | Severity | Module |
| --- | --- | --- | --- |
| 1 | P-T-V coupling missing | CRITICAL | CoupledThermo |
| 2 | Flash evaporation during outsurge | CRITICAL | PressurizerPhysics |
| 3 | Spray condensation dynamics | CRITICAL | PressurizerPhysics |
| 4 | Non-equilibrium three-region model | CRITICAL | PressurizerPhysics |
| 5 | Westinghouse reference values | CRITICAL | PlantConstants |
| 6 | Wall condensation | MAJOR | PressurizerPhysics |
| 7 | Rainout (bulk condensation) | MAJOR | PressurizerPhysics |
| 8 | Heater thermal dynamics | MAJOR | PressurizerPhysics |
| 9 | Surge line hydraulics | MAJOR | FluidFlow |
| 10 | Enthalpy transport | MAJOR | HeatTransfer |
| 11 | Steam table accuracy near saturation | MINOR | WaterProperties |
| 12 | Missing enthalpy functions | MINOR | WaterProperties |
| 13 | Steam quality/void fraction | MINOR | SteamThermo |

## 1.4 Module Structure (10 Modules, 112 Tests)

Critical/Assets/Scripts/Physics/

├── PlantConstants.cs        (5 tests)

├── WaterProperties.cs       (18 tests)

├── SteamThermodynamics.cs   (10 tests)

├── ThermalMass.cs           (6 tests)

├── ThermalExpansion.cs      (6 tests)

├── CoupledThermo.cs         (12 tests)  [GAP #1]

├── PressurizerPhysics.cs    (24 tests)  [GAPS #2-8]

├── ReactorKinetics.cs       (10 tests)

├── FluidFlow.cs             (8 tests)   [GAP #9]

├── HeatTransfer.cs          (6 tests)   [GAP #10]

└── IntegrationTests.cs      (7 tests)

# 2. REFERENCE PLANT PARAMETERS

Westinghouse 4-Loop PWR specifications for physics validation.

## 2.1 Reactor Coolant System

| Parameter | Value | Constant |
| --- | --- | --- |
| Thermal Power | 3411 MWt | THERMAL_POWER_MWT |
| RCS Water Volume | 11,500 ft³ | RCS_WATER_VOLUME |
| RCS Metal Mass | 2,200,000 lb | RCS_METAL_MASS |
| Operating Pressure | 2250 psia | OPERATING_PRESSURE |
| Thot | 619°F | T_HOT |
| Tcold | 558°F | T_COLD |
| Tavg | 588.5°F | T_AVG |
| Core ΔT | 61°F | CORE_DELTA_T |
| Total RCS Flow | 390,400 gpm | RCS_FLOW_TOTAL |

## 2.2 Pressurizer

| Parameter | Value | Constant |
| --- | --- | --- |
| Total Volume | 1800 ft³ | PZR_TOTAL_VOLUME |
| Water Volume (60%) | 1080 ft³ | PZR_WATER_VOLUME |
| Steam Volume (40%) | 720 ft³ | PZR_STEAM_VOLUME |
| Height | 52.75 ft | PZR_HEIGHT |
| Wall Mass | 200,000 lb | PZR_WALL_MASS |
| Wall Surface Area | 600 ft² | PZR_WALL_AREA |
| Total Heater Power | 1800 kW | HEATER_POWER_TOTAL |
| Heater Time Constant | 20 seconds | HEATER_TAU |
| Max Spray Flow | 900 gpm | SPRAY_FLOW_MAX |
| Spray Temperature | 558°F | (= Tcold) |

## 2.3 Pressure Setpoints

| Setpoint | Value (psig) | Constant |
| --- | --- | --- |
| Normal Operating | 2235 | P_NORMAL |
| Heaters Full ON | <2210 | P_HEATERS_ON |
| Spray ON | >2260 | P_SPRAY_ON |
| PORV Opens | 2335 | P_PORV |
| Safety Valves | 2485 | P_SAFETY |
| High Pressure Trip | 2385 | P_TRIP_HIGH |
| Low Pressure Trip | 1885 | P_TRIP_LOW |

# 3. KEY VALIDATION TESTS

## 3.1 Water Properties (18 tests)

| ID | Function | Input | Expected | Pass |
| --- | --- | --- | --- | --- |
| WP-01 | SaturationPressure | 212°F | 14.7 psia | □ |
| WP-02 | SaturationPressure | 653°F | 2250 psia | □ |
| WP-03 | SaturationTemperature | 2250 psia | 653°F | □ |
| WP-04 | WaterDensity | 588°F, 2250 psia | ~46 lb/ft³ | □ |
| WP-05 | LatentHeat | 2250 psia | ~465 BTU/lb | □ |
| WP-06 | WaterEnthalpy | 619°F, 2250 psia | ~640 BTU/lb | □ |
| WP-07 | SaturatedLiquidEnthalpy | 2250 psia | ~700 BTU/lb | □ |
| WP-08 | SubcoolingMargin | 619°F, 2250 psia | ~34°F | □ |

+ 10 additional tests for edge cases and accuracy validation

## 3.2 Coupled Thermodynamics (12 tests) - GAP #1

| ID | Test Case | Expected | Pass |
| --- | --- | --- | --- |
| CT-01 | SolveEquilibrium converges <20 iterations | converged=true | □ |
| CT-02 | 10°F rise → pressure increase | 60-80 psi | □ |
| CT-03 | Coupled expansion < uncoupled expansion | Verified | □ |
| CT-04 | Mass conserved through iteration | <0.1% error | □ |
| CT-05 | Total volume conserved | <0.01% error | □ |
| CT-06 | Steam space clamped at minimum | V≥50 ft³ | □ |

+ 6 additional tests

## 3.3 Pressurizer Physics (24 tests) - GAPS #2-8

| ID | Test Case | Expected | Pass |
| --- | --- | --- | --- |
| PZ-01 | FlashEvaporationRate at -10 psi/sec | >0 lb/sec | □ |
| PZ-02 | FlashEvaporationRate at +10 psi/sec | =0 lb/sec | □ |
| PZ-03 | SprayCondensationRate at 900 gpm | 15-30 lb/sec | □ |
| PZ-04 | WallCondensationRate when wall cold | >0 lb/sec | □ |
| PZ-05 | RainoutRate when steam subcooled | >0 lb/sec | □ |
| PZ-06 | HeaterSteamRate at 1800 kW | 3-6 lb/sec | □ |
| PZ-07 | HeaterEffectivePower lag τ≈20 sec | Verified | □ |
| PZ-08 | Flash retards pressure drop rate | ¦dP/dt¦ reduced | □ |
| PZ-09 | Spray reduces pressure rise rate | ¦dP/dt¦ reduced | □ |
| PZ-10 | Mass balance: all phase changes | Conserved | □ |

+ 14 additional tests for three-region model

## 3.4 Reactor Kinetics (10 tests)

| ID | Test Case | Expected | Pass |
| --- | --- | --- | --- |
| RK-01 | BETA_TOTAL = sum of Beta array | 0.0065 | □ |
| RK-02 | Doppler at hot fuel (2000°F) | <0 pcm | □ |
| RK-03 | MTC at high boron (1500 ppm) | ~+5 pcm/°F | □ |
| RK-04 | MTC at low boron (100 ppm) | ~-40 pcm/°F | □ |
| RK-05 | Subcritical: power decays | Decreasing | □ |
| RK-06 | Critical: power steady | Constant | □ |

+ 4 additional tests

## 3.5 Fluid Flow (8 tests) - GAP #9

| ID | Test Case | Expected | Pass |
| --- | --- | --- | --- |
| FF-01 | 4 RCPs at full speed | 390,400 gpm | □ |
| FF-02 | Coastdown at t=τ (12 sec) | 37% initial | □ |
| FF-03 | Natural circ at ΔT=61°F | 3-6% flow | □ |
| FF-04 | SurgeLineFlow at ΔP=+10 psi | >0 (insurge) | □ |
| FF-05 | SurgeLineFlow at ΔP=-10 psi | <0 (outsurge) | □ |

+ 3 additional tests

## 3.6 Heat Transfer (6 tests) - GAP #10

| ID | Test Case | Expected | Pass |
| --- | --- | --- | --- |
| HT-01 | SurgeEnthalpyDeficit at 619°F | 50-70 BTU/lb | □ |
| HT-02 | SurgeEnthalpyDeficit at Tsat | ~0 BTU/lb | □ |
| HT-03 | SurgeHeatingLoad at 500 gpm | >0 BTU/sec | □ |
| HT-04 | CondensingHTC at 2250 psia | ~200 | □ |

+ 2 additional tests

## 3.7 Integration Tests (7 tests)

| ID | Test Case | Expected | Pass |
| --- | --- | --- | --- |
| INT-01 | 10°F Tavg rise: coupled pressure response | 60-80 psi | □ |
| INT-02 | Insurge: pressure + flash + spray | Realistic | □ |
| INT-03 | Outsurge: flash retards pressure drop | Verified | □ |
| INT-04 | Power trip: Doppler + MTC feedback | P decreases | □ |
| INT-05 | RCP coastdown + nat circ transition | Smooth | □ |
| INT-06 | Mass conservation full transient | <0.1% err | □ |
| INT-07 | Energy conservation full transient | <1% err | □ |

# 4. PHASE 1 EXIT GATE

ALL criteria must be met before proceeding to Phase 2.

## 4.1 Test Requirements

| Requirement | Target | Status |
| --- | --- | --- |
| All 112 unit tests passing | 112/112 | □ |
| No tests skipped | 0 skipped | □ |
| Test execution <10 seconds | <10 sec | □ |
| All integration tests passing | 7/7 | □ |

## 4.2 Physics Validation

| Validation | Criterion | Status |
| --- | --- | --- |
| P-T-V solver converges | <20 iter | □ |
| 10°F rise → pressure increase | 60-80 psi | □ |
| Flash rate matches literature | ±30% | □ |
| Spray rate realistic | ±30% | □ |
| Subcooling always positive | >0°F | □ |
| Mass conserved | <0.1% | □ |
| Energy conserved | <1% | □ |

## 4.3 Code Review

| Review Item | Status |
| --- | --- |
| No MonoBehaviours in Physics namespace | □ |
| No magic numbers (all in PlantConstants) | □ |
| No Unity dependencies except Mathf | □ |
| All divisions protected against zero | □ |
| Consistent units (°F, psia, ft³, lb, BTU) | □ |
| All public methods documented | □ |

# APPENDIX: TEST SUMMARY

| Module | Tests | Test File |
| --- | --- | --- |
| PlantConstants | 5 | PlantConstantsTests.cs |
| WaterProperties | 18 | WaterPropertiesTests.cs |
| SteamThermodynamics | 10 | SteamThermodynamicsTests.cs |
| ThermalMass | 6 | ThermalMassTests.cs |
| ThermalExpansion | 6 | ThermalExpansionTests.cs |
| CoupledThermo | 12 | CoupledThermoTests.cs |
| PressurizerPhysics | 24 | PressurizerPhysicsTests.cs |
| ReactorKinetics | 10 | ReactorKineticsTests.cs |
| FluidFlow | 8 | FluidFlowTests.cs |
| HeatTransfer | 6 | HeatTransferTests.cs |
| Integration | 7 | IntegrationTests.cs |
| TOTAL | 112 | - |

Phase 1 establishes the physics foundation. All subsequent phases depend on these calculations.

Estimated Duration: 3-4 weeks

Next: Phase 2 - Reactor Core Implementation