CRITICAL

MASTER THE ATOM

PHASE 2

REACTOR CORE

Fuel, Control Rods, and Coolant Integration

Duration: 3-4 weeks  |  Unit Tests: 85  |  Prerequisites: Phase 1 Complete

# 1. PHASE OVERVIEW

## 1.1 Purpose

Phase 2 implements the reactor core model using the Phase 1 physics library. This phase creates the fuel assemblies, control rods, and coolant flow model that form the heart of the reactor simulation.

Phase 2 introduces Unity GameObjects - the first MonoBehaviours in the project.

## 1.2 Prerequisites from Phase 1

Phase 2 depends on these Phase 1 modules being complete and tested:

• WaterProperties - for coolant thermodynamic properties

• ReactorKinetics - for neutron kinetics, Doppler, MTC, boron feedback

• ThermalMass - for fuel and coolant heat capacity

• FluidFlow - for RCP flow calculations

• CoupledThermo - for pressure-temperature coupling during transients

• PlantConstants - for Westinghouse 4-loop reference values

## 1.3 Key Components

| Component | Description | Tests |
| --- | --- | --- |
| FuelAssembly | 193 assemblies, 264 rods each, fuel temp model | 15 |
| ControlRodBank | 8 banks (A,B,C,D,SA,SB,SC,SD), rod worth curves | 12 |
| CoreCoolant | 4-loop flow model, Thot/Tcold calculation | 10 |
| ReactorCore | Integration: kinetics + thermal + hydraulics | 18 |
| CoreController | Unity MonoBehaviour, Update loop, events | 8 |
| PowerCalculator | Thermal power from neutron flux | 6 |
| FeedbackCalculator | Doppler + MTC + boron integration | 8 |
| Integration | End-to-end reactor transient tests | 8 |

## 1.4 Project Structure

Critical/Assets/Scripts/

├── Physics/                 ← Phase 1 (complete)

└── Core/                    ← THIS PHASE

├── FuelAssembly.cs

├── ControlRodBank.cs

├── CoreCoolant.cs

├── ReactorCore.cs

├── CoreController.cs    ← First MonoBehaviour

├── PowerCalculator.cs

└── FeedbackCalculator.cs

# 2. REACTOR CORE REFERENCE VALUES

## 2.1 Westinghouse 4-Loop Core

| Parameter | Value |
| --- | --- |
| Number of Fuel Assemblies | 193 |
| Fuel Rods per Assembly | 264 |
| Total Fuel Rods | 50,952 |
| Active Fuel Height | 12 feet |
| Core Diameter | ~11.1 feet |
| Average Linear Heat Rate | 5.44 kW/ft |
| Peak Linear Heat Rate | ~13 kW/ft (max) |
| Average Fuel Temperature | ~1200°F |
| Peak Fuel Centerline | ~3800°F (limit 4700°F) |

## 2.2 Control Rod Configuration

| Bank | Type | Assemblies | Worth (pcm) |
| --- | --- | --- | --- |
| D | Control | 8 | ~800 |
| C | Control | 8 | ~700 |
| B | Control | 8 | ~700 |
| A | Control | 8 | ~700 |
| SA | Shutdown | 8 | ~1000 |
| SB | Shutdown | 8 | ~1000 |
| SC | Shutdown | 8 | ~1000 |
| SD | Shutdown | 8 | ~1000 |

Total shutdown margin with all rods inserted: ~8000 pcm

Rod speed: 72 steps/minute (228 steps total travel)

## 2.3 Reactivity Coefficients

| Coefficient | Value | Source |
| --- | --- | --- |
| Doppler (αD) | -2.5 pcm/√°R | Phase 1 |
| MTC (high boron) | +5 pcm/°F | Phase 1 |
| MTC (low boron) | -40 pcm/°F | Phase 1 |
| Boron Worth | -9 pcm/ppm | Phase 1 |
| Delayed Neutron Fraction | 0.0065 | Phase 1 |

# 3. VALIDATION TESTS (85 Total)

## 3.1 FuelAssembly Tests (15)

| ID | Test Case | Expected | Pass |
| --- | --- | --- | --- |
| FA-01 | Initialize 193 assemblies | Count=193 | □ |
| FA-02 | Fuel temp rises with power | dT/dP > 0 | □ |
| FA-03 | Average fuel temp at 100% power | ~1200°F | □ |
| FA-04 | Peak centerline at 100% power | <4700°F | □ |
| FA-05 | Fuel temp at 0% power | = Tcoolant | □ |
| FA-06 | Heat transfer to coolant | Q = P_thermal | □ |
| FA-07 | Fuel thermal time constant | ~5-10 sec | □ |

+ 8 additional tests for fuel dynamics and edge cases

## 3.2 ControlRodBank Tests (12)

| ID | Test Case | Expected | Pass |
| --- | --- | --- | --- |
| CR-01 | Bank D fully withdrawn | Step=228 | □ |
| CR-02 | Bank D fully inserted | Step=0 | □ |
| CR-03 | Rod worth follows integral curve | Non-linear | □ |
| CR-04 | Rod speed = 72 steps/min | Verified | □ |
| CR-05 | Trip: all rods to bottom | <2 seconds | □ |
| CR-06 | Shutdown margin with all in | ~8000 pcm | □ |

+ 6 additional tests for bank sequencing and interlocks

## 3.3 CoreCoolant Tests (10)

| ID | Test Case | Expected | Pass |
| --- | --- | --- | --- |
| CC-01 | Thot at 100% power, 100% flow | 619°F | □ |
| CC-02 | Tcold at 100% power, 100% flow | 558°F | □ |
| CC-03 | Core ΔT at 100% power | 61°F | □ |
| CC-04 | ΔT proportional to P/flow | Linear | □ |
| CC-05 | 4-loop flow distribution | Equal ±5% | □ |
| CC-06 | Loss of 1 RCP: flow reduction | ~75% | □ |

+ 4 additional tests for flow transients

## 3.4 ReactorCore Integration Tests (18)

| ID | Test Case | Expected | Pass |
| --- | --- | --- | --- |
| RC-01 | Steady state at 100% power | Stable ±1% | □ |
| RC-02 | Rod withdrawal: power increases | Rising | □ |
| RC-03 | Rod insertion: power decreases | Falling | □ |
| RC-04 | Doppler limits power excursion | Self-limiting | □ |
| RC-05 | MTC feedback on temp change | Correct sign | □ |
| RC-06 | Boron dilution: power rises | Slow rise | □ |
| RC-07 | Boron addition: power drops | Slow drop | □ |
| RC-08 | Reactor trip: power drops fast | <5% in 2s | □ |
| RC-09 | Coolant temp couples to PZR | P responds | □ |
| RC-10 | Loss of flow: temps rise | ΔT increases | □ |

+ 8 additional integration tests

## 3.5 Other Module Tests

| Module | Tests | Key Validations |
| --- | --- | --- |
| CoreController | 8 | Unity Update loop, event dispatch, state machine |
| PowerCalculator | 6 | Thermal power from flux, decay heat |
| FeedbackCalculator | 8 | Combined Doppler+MTC+boron reactivity |
| Integration | 8 | Full transient scenarios |

# 4. PHASE 2 EXIT GATE

ALL criteria must be met before proceeding to Phase 3.

## 4.1 Test Requirements

| Requirement | Target | Status |
| --- | --- | --- |
| All 85 unit tests passing | 85/85 | □ |
| Phase 1 tests still passing | 112/112 | □ |
| No regressions in physics library | Verified | □ |
| Unity Play mode tests pass | All pass | □ |

## 4.2 Physics Validation

| Validation | Criterion | Status |
| --- | --- | --- |
| Steady state power stable | ±1% for 60s | □ |
| Thot/Tcold match reference | 619/558°F ±2 | □ |
| Rod worth curves validated | Within ±10% | □ |
| Doppler feedback self-limiting | Verified | □ |
| Reactor trip <2 seconds | P<5% | □ |
| Core couples to pressurizer | ΔT → ΔP | □ |

## 4.3 Test Summary

| Module | Tests | Test File |
| --- | --- | --- |
| FuelAssembly | 15 | FuelAssemblyTests.cs |
| ControlRodBank | 12 | ControlRodBankTests.cs |
| CoreCoolant | 10 | CoreCoolantTests.cs |
| ReactorCore | 18 | ReactorCoreTests.cs |
| CoreController | 8 | CoreControllerTests.cs |
| PowerCalculator | 6 | PowerCalculatorTests.cs |
| FeedbackCalculator | 8 | FeedbackCalculatorTests.cs |
| Integration | 8 | CoreIntegrationTests.cs |
| TOTAL | 85 | - |

Phase 2 creates the reactor core model. Automatic stabilization via inherent feedback (Doppler, MTC).

Duration: 3-4 weeks

Next: Phase 3 - Pressurizer System