CRITICAL

MASTER THE ATOM

PHASE 3: PRESSURIZER SYSTEM

Complete Implementation with Phase 1 Physics Integration

Duration: 3-4 weeks | Tests: 54 Unit + 24 Integration = 78 Total

# 1. PHASE OVERVIEW

Phase 3 replaces AUTO pressurizer with physics-based model using Phase 1 library.

Implements 9 of 13 Gap Analysis fixes (Gaps #1-4, #6-10). First system with real dynamics.

## 1.1 Entry Criteria

✓ Phase 1 Gate PASSED (112/112 physics tests)

✓ Phase 2 Gate PASSED (85/85 reactor core tests)

## 1.2 System Status After Phase 3

| System | Mode | Notes |
| --- | --- | --- |
| Reactor Core | MANUAL | Phase 2 |
| Pressurizer | MANUAL | THIS PHASE |
| SGs/CVCS/RCPs/Turbine | AUTO | Boundary conditions |

# 2. PHASE 1 PHYSICS MODULE DEPENDENCIES

| Phase 1 Module | Phase 3 Usage | Gap # |
| --- | --- | --- |
| PlantConstants.cs | PZR: 1800 ft³, 200,000 lb wall, setpoints | #5 |
| WaterProperties.cs | Tsat=653°F @ 2250 psia, h=640/700 BTU/lb | #11,12 |
| SteamThermodynamics.cs | Void fraction, steam quality | #13 |
| ThermalMass.cs | Wall: 200,000 lb × 0.12 BTU/lb·°F | - |
| ThermalExpansion.cs | RCS surge volume from ΔTavg | - |
| CoupledThermo.cs | ALL pressure calcs - iterative solver | #1 |
| PressurizerPhysics.cs | Flash, spray, wall, rainout, heater | #2-4,6-8 |
| FluidFlow.cs | Surge line: 14" × 50 ft, f=0.015 | #9 |
| HeatTransfer.cs | Enthalpy deficit: ~60 BTU/lb | #10 |

## 2.1 Module Call Sequence (Every Timestep)

1. ThermalExpansion.SurgeVolume(11500, ΔTavg, P)

2. FluidFlow.SurgeLineFlow(ΔP, 14", 50ft)     [GAP #9]

3. HeatTransfer.EnthalpyTransport(flow, 640, 700) [GAP #10]

4. PressurizerPhysics.ThreeRegionModel(state)  [GAP #4]

5. CoupledThermo.SolveEquilibrium(state)       [GAP #1] ← ITERATIVE

6-10. Flash/Heater/Spray/Wall/Rainout          [GAPs #2,3,6,7,8]

CRITICAL: Never bypass CoupledThermo - simple PV^n gives WRONG answers.

# 3. GAP ANALYSIS IMPLEMENTATION

| Gap | Issue | Without | With (Validated) |
| --- | --- | --- | --- |
| #1 | P-T-V coupling | 10°F→0 psi | 10°F→60-80 psi (EPRI) |
| #2 | Flash evaporation | Rapid drop | Self-regulating (RELAP5) |
| #3 | Spray condensation | Instant 100% | η=85% (EPRI: 80-95%) |
| #4 | Three-region | 2-phase | Subcooled/sat/steam |
| #6 | Wall condensation | None | 200,000 lb steel |
| #7 | Rainout | None | When Tsteam<Tsat |
| #8 | Heater lag | Instant | τ=20s (15-25s typ) |
| #9 | Surge line | No resist | Darcy-Weisbach |
| #10 | Enthalpy | None | ~60 BTU/lb (NIST) |

# 4. PRESSURIZER REFERENCE PARAMETERS

All values validated against Westinghouse 4-loop FSAR and NIST Steam Tables.

## 4.1 Physical Dimensions

| Parameter | Value | Source |
| --- | --- | --- |
| Total Volume | 1800 ft³ | FSAR |
| Water Volume (60%) | 1080 ft³ | Normal ops |
| Steam Volume (40%) | 720 ft³ | Normal ops |
| Wall Mass | 200,000 lb | SA-533B steel |
| Height | 52.75 ft | FSAR |
| Tsat @ 2250 psia | 653°F | NIST |

## 4.2 Heater System

| Parameter | Value | Notes |
| --- | --- | --- |
| Total Power | 1800 kW | 5 groups |
| Proportional | 500 kW | Modulating |
| Backup | 1300 kW | ON/OFF |
| Time Constant τ | 20 seconds | Industry: 15-25s |
| 63% response | @ 20s (1τ) | First-order |
| 95% response | @ 60s (3τ) | Near full |

## 4.3 Spray System

| Parameter | Value | Notes |
| --- | --- | --- |
| Max Flow | 900 gpm | Both valves |
| Temperature | 558°F (=Tcold) | From cold leg |
| Efficiency η | 85% | EPRI: 80-95% |
| Steam condensed | 15-25 lb/s | @ 900 gpm |

## 4.4 Enthalpy Transport (Gap #10)

| Parameter | Value | Source |
| --- | --- | --- |
| Surge enthalpy (619°F) | ~640 BTU/lb | NIST |
| Sat liquid enthalpy (653°F) | ~700 BTU/lb | NIST |
| Enthalpy deficit | ~60 BTU/lb | 700-640 |
| Subcooling margin | 34°F | 653-619 |

## 4.5 Pressure Setpoints

| Setpoint | Pressure | Action |
| --- | --- | --- |
| Spray Full ON | >2280 psig | 900 gpm spray |
| Spray ON | >2260 psig | Proportional spray |
| Normal | 2235 psig | Proportional heaters |
| Heaters Full ON | <2210 psig | All 1800 kW |
| PORV Opens | >2335 psig | Relief valve lifts |
| Safety Valve | >2485 psig | Code safety opens |
| High Trip | >2385 psig | Reactor trip |
| Low Trip | <1885 psig | Reactor trip |

# 5. UNIT TEST SUITE (54 Tests)

## 5.1 Phase 1 Integration Tests (24 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| P1-01 | CoupledThermo.SolveEquilibrium converges <20 iter | converged | □ |
| P1-02 | 10°F Tavg rise → 60-80 psi increase (NOT 0) | 60-80 psi | □ |
| P1-03 | FlashEvaporation positive on depressurization | >0 lb/s | □ |
| P1-04 | SprayCondensation @ 900 gpm, η=85% | 15-25 lb/s | □ |
| P1-05 | WallCondensation continuous at steady state | >0 lb/s | □ |
| P1-06 | Rainout when Tsteam < Tsat | >0 lb/s | □ |
| P1-07 | HeaterSteamGen with τ=20s (63% @ 20s) | 63% | □ |
| P1-08 | ThreeRegionModel tracks subcooled mass | >0 lb | □ |
| P1-09 | SurgeLineFlow proportional to √ΔP | √ΔP | □ |
| P1-10 | EnthalpyTransport negative for insurge | <0 BTU/s | □ |
| P1-11 | Enthalpy deficit ~60 BTU/lb | 55-65 | □ |
| P1-12 | Tsat(2250)=653°F, hfg=465 BTU/lb | NIST | □ |

+ 12 additional Phase 1 integration tests (P1-13 to P1-24)

## 5.2 Pressurizer State Tests (12 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| PZ-01 | Level = liquidVol/1800×100; 60%→1080 ft³ | Formula | □ |
| PZ-02 | Mass conservation: total mass constant | <0.1% err | □ |
| PZ-03 | Energy conservation validated | <1% err | □ |
| PZ-04 | Surge IN→level↑, OUT→level↓ | Correct | □ |
| PZ-05 | Heater ON→P↑, Spray ON→P↓ | Correct | □ |
| PZ-06 | Steam bubble maintained (V>0 always) | V_s > 0 | □ |

+ 6 additional state tests (PZ-07 to PZ-12)

## 5.3 Control System Tests (10 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| CT-01 | Heaters OFF @ P>2235, Full @ P<2210 | 0/1800 kW | □ |
| CT-02 | Spray OFF @ P<2260, Full @ P>2280 | 0/900 gpm | □ |
| CT-03 | Heaters/spray mutually exclusive | Never both | □ |
| CT-04 | Pressure returns to 2235±5 after transient | 2235±5 | □ |

+ 6 additional control tests (CT-05 to CT-10)

## 5.4 Relief Valve Tests (8 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| RV-01 | PORV opens@2335, closes@2300 (hysteresis) | 35 psi band | □ |
| RV-02 | PORV capacity ~210,000 lb/hr | ~58 lb/s | □ |
| RV-03 | Safety@2485, reseats@~2400 | 85 psi band | □ |
| RV-04 | Safety capacity ~400,000 lb/hr each | ~111 lb/s | □ |

+ 4 additional relief valve tests (RV-05 to RV-08)

# 6. INTEGRATION TEST SUITE (24 Tests)

## 6.1 Transient Response Tests (12 tests)

| ID | Scenario | Criteria | □ |
| --- | --- | --- | --- |
| INT-301 | 10% power increase | P spike ~20 psi, recover <60s | □ |
| INT-302 | 10% power decrease | P drop ~20 psi, recover <60s | □ |
| INT-303 | Reactor trip from 100% | P spike then heaters recover | □ |
| INT-304 | 100% load rejection | No PORV lift (<2335) | □ |
| INT-305 | Stuck open PORV | Rapid depress, flash visible | □ |
| INT-306 | 10°F Tavg rise insurge | P rises 60-80 psi (Gap #1) | □ |

+ 6 additional transient tests (INT-307 to INT-312)

## 6.2 Steady State Tests (6 tests)

| ID | Condition | Tolerance | □ |
| --- | --- | --- | --- |
| SS-01 | Pressure at 100% power | 2235 ± 5 psig | □ |
| SS-02 | Level at 100% power | 60 ± 3% | □ |
| SS-03 | Heater power at equilibrium | ~300 kW (losses) | □ |
| SS-04 | Pressure drift rate | < 5 psi/hour | □ |

+ 2 additional steady state tests (SS-05 to SS-06)

## 6.3 Coupling Validation Tests (6 tests)

Validates all Phase 1 physics gaps correctly implemented.

# 7. PHASE 3 EXIT GATE

| Requirement | Status | Date |
| --- | --- | --- |
| All 54 unit tests pass (24+12+10+8) | □ PASS | ________ |
| All 24 integration tests pass | □ PASS | ________ |
| 10°F→60-80 psi verified (Gap #1) | □ PASS | ________ |
| Heater τ=20s verified (Gap #8) | □ PASS | ________ |
| Flash retards P drop (Gap #2) | □ PASS | ________ |
| Spray η=85% verified (Gap #3) | □ PASS | ________ |
| Enthalpy deficit ~60 BTU/lb (Gap #10) | □ PASS | ________ |
| Steam bubble maintained always | □ PASS | ________ |
| Phase 1 tests still passing (112/112) | □ PASS | ________ |
| Phase 2 tests still passing (85/85) | □ PASS | ________ |
| PHASE 3 GATE APPROVED | □ YES | ________ |

## 7.1 Test Summary

| Category | Count | File |
| --- | --- | --- |
| Phase 1 Integration | 24 | Phase1IntegrationTests.cs |
| Pressurizer State | 12 | PressurizerStateTests.cs |
| Control System | 10 | PressureControlTests.cs |
| Relief Valves | 8 | ReliefValveTests.cs |
| Unit Subtotal | 54 | - |
| Integration | 24 | PressurizerIntegrationTests.cs |
| PHASE 3 TOTAL | 78 | - |

Phase 3 Gate Passed → Proceed to Phase 4: CVCS & RCPs