CRITICAL

MASTER THE ATOM

PHASE 4: CVCS & RCPs

Boron Transport Delay & Pump Dynamics

Duration: 2-3 weeks | Tests: 41 Unit + 24 Integration = 65 Total

# 1. PHASE OVERVIEW

Phase 4 adds CVCS (boron with 5-15 min transport delay) and RCPs (coastdown τ=12s, natural circulation).

Key modules: FluidFlow.cs (PumpCoastdown, NaturalCirculation), ReactorKinetics.cs (BoronReactivity).

## 1.1 Entry Criteria

✓ Phase 3 Gate PASSED (pressurizer physics working)

✓ Phase 1-2 tests still passing (112+85=197 tests)

## 1.2 System Status After Phase 4

| System | Mode | Notes |
| --- | --- | --- |
| Reactor Core | MANUAL | Phase 2 |
| Pressurizer | MANUAL | Phase 3 |
| CVCS | MANUAL | THIS PHASE - Boron delay |
| RCPs (4) | MANUAL | THIS PHASE - Coastdown |
| SGs/Turbine | AUTO | Boundary conditions |

# 2. PHASE 1 PHYSICS DEPENDENCIES

| Phase 1 Module | Phase 4 Usage |
| --- | --- |
| PlantConstants.cs | RCP: 97,600 gpm/pump, 1189 rpm, τ=12s; CVCS flows |
| FluidFlow.cs | PumpCoastdown(), AffinityLaws(), NaturalCirculation() |
| ReactorKinetics.cs | BoronReactivity() → -9 pcm/ppm |
| WaterProperties.cs | LiquidDensity() for boron concentration |
| HeatTransfer.cs | RCP heat addition: 21 MW per pump (84 MW total) |

## 2.1 Boron Transport Delay

Boron changes take 5-15 minutes (~10 min typical) to reach core.

const float TRANSPORT_TIME = 600f;  // 10 minutes

// Ring buffer tracks boron packets in transit

state.boronPipeline = new Queue<BoronPacket>();

Without delay: Dilution → INSTANT reactivity → unrealistic

With delay: Operator has ~10 min to respond to boron changes

Boron worth: -9 pcm/ppm (100 ppm dilution = ~900 pcm reactivity)

## 2.2 RCP Coastdown

Exponential coastdown with τ = 12 seconds (Westinghouse 93A pumps).

// speed(t) = speed₀ × e^(-t/τ)

// At t=τ (12s): 37% of initial (e⁻¹ = 0.368)

// At t=3τ (36s): 5% of initial

pump.speed = FluidFlow.PumpCoastdown(pump.speed, dt, 12f);

pump.flow = FluidFlow.AffinityLaws_Flow(pump.speed, 1189, 97600);

## 2.3 Natural Circulation (CORRECTED)

After all RCPs trip, natural circulation provides 12,000-23,000 gpm (3-6% of normal).

// Driving force: density difference between hot and cold legs

float dRho = WaterProperties.Density(558) - WaterProperties.Density(619);

float head = dRho * CORE_HEIGHT * g;  // ~0.5 psi

float flow = FluidFlow.NaturalCirculation(head, resistance);

Natural circulation flow: 12,000-23,000 gpm (FSAR validated)

This is 3-6% of normal 390,400 gpm flow

Core ΔT increases from 61°F to >100°F at reduced flow

# 3. REFERENCE PARAMETERS

## 3.1 CVCS Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Normal Letdown | 75 gpm | To VCT |
| Normal Charging | 87 gpm total | 55 RCS + 32 seals |
| Seal Injection | 32 gpm | 8 gpm × 4 pumps |
| Boric Acid Conc | 7000 ppm | In BA tank |
| Boron Worth | -9 pcm/ppm | ReactorKinetics |
| Transport Time | 5-15 min (~10 typ) | VCT→Core delay |

## 3.2 RCP Parameters

| Parameter | Value | Source |
| --- | --- | --- |
| Number of Pumps | 4 | One per loop |
| Flow per Pump | 97,600 gpm | Westinghouse 93A |
| Total RCS Flow | 390,400 gpm | 4 × 97,600 |
| Nominal Speed | 1189 rpm | FSAR |
| Coastdown τ | 12 seconds | FluidFlow.cs |
| Heat Addition | 21 MW/pump | 84 MW total |
| Low Flow Trip | 87% | Reactor trip |
| Natural Circ Flow | 12,000-23,000 gpm | 3-6% of normal |

# 4. UNIT TEST SUITE (41 Tests)

## 4.1 CVCS Boron Tests (20 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| CV-01 | Boron transport delay measurable | 5-15 min | □ |
| CV-02 | Typical transport ~10 min | ~600s | □ |
| CV-03 | 100 ppm dilution = ~900 pcm | 900 pcm | □ |
| CV-04 | BoronReactivity = -9 pcm/ppm | -9 | □ |
| CV-05 | Ring buffer FIFO ordering | Correct | □ |
| CV-06 | Boron mass conservation | <0.1% err | □ |
| CV-07 | High boron (1500 ppm) → MTC=+5 pcm/°F | +5 | □ |
| CV-08 | Low boron (100 ppm) → MTC=-40 pcm/°F | -40 | □ |

+ 12 additional CVCS tests (CV-09 to CV-20)

## 4.2 RCP Tests (21 tests)

| ID | Test Case | Expected | □ |
| --- | --- | --- | --- |
| RCP-01 | Coastdown τ = 12 ± 3 seconds | 9-15s | □ |
| RCP-02 | Speed @ t=τ (12s) = 37% | 37% | □ |
| RCP-03 | Speed @ t=3τ (36s) = 5% | 5% | □ |
| RCP-04 | 1 pump trip → ~75% flow | ~75% | □ |
| RCP-05 | Affinity: Q ∝ N, H ∝ N² | Laws | □ |
| RCP-06 | Low flow trip @ <87% | Trip | □ |
| RCP-07 | 4 RCPs = 390,400 gpm | 390,400 | □ |
| RCP-08 | RCP heat = 21 MW each | 21 MW | □ |
| RCP-09 | Natural circ = 12,000-23,000 gpm (3-6%) | 12-23k | □ |
| RCP-10 | Nat circ driven by ΔT density diff | dρ×h×g | □ |

+ 11 additional RCP tests (RCP-11 to RCP-21)

# 5. INTEGRATION TEST SUITE (24 Tests)

## 5.1 CVCS Transients (12 tests)

| ID | Scenario | Criteria | □ |
| --- | --- | --- | --- |
| INT-401 | Dilution at 30 gpm: delayed power rise | Delay ~10 min visible | □ |
| INT-402 | Boration at 30 gpm: delayed decrease | Delay ~10 min visible | □ |
| INT-403 | 100 ppm dilution → power effect | ~900 pcm effect | □ |
| INT-404 | Emergency boration recovery | Recover in ~15 min | □ |

+ 8 additional CVCS integration tests

## 5.2 RCP Transients (12 tests)

| ID | Scenario | Criteria | □ |
| --- | --- | --- | --- |
| INT-405 | Single RCP trip at 100% | Flow drops, ΔT up | □ |
| INT-406 | 2 RCP trip → reactor trip (<87%) | Rx trip | □ |
| INT-407 | Loss of all RCPs: nat circ transition | 12-23k gpm achieved | □ |
| INT-408 | Nat circ ΔT increases | ΔT > 100°F | □ |
| INT-409 | Coastdown matches τ=12s | 37% @ 12s | □ |

+ 7 additional RCP integration tests

# 6. PHASE 4 EXIT GATE

| Requirement | Status | Date |
| --- | --- | --- |
| Boron transport delay 5-15 min verified | □ PASS | ________ |
| 100 ppm = ~900 pcm verified | □ PASS | ________ |
| RCP coastdown τ=12±3s verified | □ PASS | ________ |
| 1 RCP trip → ~75% flow | □ PASS | ________ |
| Low flow trip (<87%) functions | □ PASS | ________ |
| Natural circ 12,000-23,000 gpm (3-6%) | □ PASS | ________ |
| All 41 unit tests pass | □ PASS | ________ |
| All 24 integration tests pass | □ PASS | ________ |
| Phase 1-3 tests still passing | □ PASS | ________ |
| PHASE 4 GATE APPROVED | □ YES | ________ |

## 6.1 Test Summary

| Category | Count | File |
| --- | --- | --- |
| CVCS Boron Control | 20 | CVCSBoronTests.cs |
| RCP Dynamics | 21 | RCPDynamicsTests.cs |
| Unit Subtotal | 41 | - |
| Integration | 24 | Phase4IntegrationTests.cs |
| PHASE 4 TOTAL | 65 | - |

Phase 4 Gate Passed → Proceed to Phase 5: Steam Generators