CRITICAL

MASTER THE ATOM

PHASE 5: STEAM GENERATORS

Heat Transfer, Shrink/Swell & Level Control

Duration: 3-4 weeks | Tests: 48 Unit + 24 Integration = 72 Total

# 1. PHASE OVERVIEW

Phase 5 closes the thermal loop: reactor heat must be actively removed by SGs.

Key modules: HeatTransfer.cs (LMTD, UA), WaterProperties.cs (secondary saturation).

## 1.1 Critical Coupling

SGs couple primary/secondary: Less heat removal → RCS heats up → pressure rises

## 1.2 System Status

| System | Mode | Notes |
| --- | --- | --- |
| Reactor/PZR/CVCS/RCPs | MANUAL | Phases 2-4 |
| Steam Generators | MANUAL | THIS PHASE |
| Turbine/Generator | AUTO | Phase 6 |

# 2. PHASE 1 PHYSICS DEPENDENCIES

| Module | Usage |
| --- | --- |
| PlantConstants.cs | SG: 55,000 ft² area, 5,626 tubes, Model F |
| WaterProperties.cs | Tsat(1000 psia)=545°F secondary |
| HeatTransfer.cs | LMTD(), UA for primary-secondary transfer |
| SteamThermodynamics.cs | Void fraction for shrink/swell |

## 2.1 LMTD Calculation (CORRECTED)

ΔT1 = Thot - Tsteam = 619 - 545 = 74°F  (hot end)

ΔT2 = Tcold - Tsteam = 558 - 545 = 13°F  (cold end)

LMTD = (74-13) / ln(74/13) = 61/1.74 ≈ 35°F

With correction factor: LMTD ≈ 43°F at 100% power

## 2.2 Shrink/Swell

Level moves OPPOSITE to expectation initially:

SHRINK: Power↑ → voids↑ → level DROPS (mass unchanged)

SWELL: Power↓ → voids↓ → level RISES (mass unchanged)

# 3. STEAM GENERATOR PARAMETERS

| Parameter | Value | Source |
| --- | --- | --- |
| Number of SGs | 4 | One/loop |
| Heat Transfer Area | 55,000 ft² each | FSAR |
| U-tubes | 5,626 | Model F |
| Steam Flow (100%) | 3.8×10⁶ lb/hr each | 15.2M total |
| Steam Pressure | 1000 psia | 100% power |
| Steam Temp | 545°F | Sat@1000 |
| Feedwater Temp | 440°F | After heaters |
| LMTD @ 100% | ~43°F | Calculated |

# 4. UNIT TEST SUITE (48 Tests)

## 4.1 Heat Transfer (16 tests)

| ID | Test | Expected | □ |
| --- | --- | --- | --- |
| HT-01 | Heat transfer = power at steady state | 3411 MW | □ |
| HT-02 | LMTD calculated correctly | ~43°F | □ |
| HT-03 | Tcold from heat balance | 558±2°F | □ |
| HT-04 | Q = U×A×LMTD verified | Formula | □ |

+ 12 additional heat transfer tests

## 4.2 Shrink/Swell (12 tests)

| ID | Test | Expected | □ |
| --- | --- | --- | --- |
| SS-01 | SHRINK: Power↑ → level DROPS | Drops | □ |
| SS-02 | SWELL: Power↓ → level RISES | Rises | □ |
| SS-03 | Recovery in 2-3 minutes | 2-3 min | □ |
| SS-04 | Mass unchanged during shrink/swell | Δm≈0 | □ |

+ 8 additional shrink/swell tests

## 4.3 Level/Pressure Control (20 tests)

3-element FW control, steam dump, MSIV, level trips

# 5. INTEGRATION TESTS (24 Tests)

| ID | Scenario | Criteria | □ |
| --- | --- | --- | --- |
| INT-501 | 10% load increase | P dip ~30 psi, recover | □ |
| INT-502 | Turbine trip | Steam P→~1050, no safety | □ |
| INT-503 | Shrink on power increase | Recover 2-3 min | □ |
| INT-504 | LO-LO level | Reactor trip | □ |
| INT-505 | HI-HI level | Turbine trip | □ |
| INT-506 | Cooldown at 50°F/hr | Controlled | □ |

+ 18 additional integration tests

# 6. PHASE 5 EXIT GATE

| Requirement | Status | Date |
| --- | --- | --- |
| Heat transfer = 3411 MW | □ PASS | ________ |
| LMTD ~43°F verified | □ PASS | ________ |
| Shrink/swell validated | □ PASS | ________ |
| Steam P stable 1000±20 psia | □ PASS | ________ |
| Level 50±5% maintained | □ PASS | ________ |
| All 72 tests pass | □ PASS | ________ |
| Phases 1-4 still passing | □ PASS | ________ |
| PHASE 5 GATE APPROVED | □ YES | ________ |

## 6.1 Test Summary

| Category | Count | File |
| --- | --- | --- |
| Heat Transfer | 16 | HeatTransferTests.cs |
| Shrink/Swell | 12 | ShrinkSwellTests.cs |
| Control | 20 | SGControlTests.cs |
| Unit Subtotal | 48 | - |
| Integration | 24 | Phase5IntegrationTests.cs |
| PHASE 5 TOTAL | 72 | - |

Phase 5 Gate Passed → Proceed to Phase 6: Turbine/Generator