CRITICAL

MASTER THE ATOM

PHASE 6: TURBINE/GENERATOR

Complete Plant Integration - ALL Systems MANUAL

Duration: 2-3 weeks | Tests: 44 Unit + 24 Integration = 68 Total

# 1. PHASE OVERVIEW

Phase 6 completes Critical. ALL systems are now MANUAL - full energy chain controlled by operator.

Energy chain: Reactor(fission) → Coolant → SGs → Steam → Turbine → Generator → Grid

## 1.1 Final System Status

| System | Mode | Phase |
| --- | --- | --- |
| Reactor Core | MANUAL | Phase 2 |
| Pressurizer | MANUAL | Phase 3 |
| CVCS | MANUAL | Phase 4 |
| RCPs (4) | MANUAL | Phase 4 |
| Steam Generators (4) | MANUAL | Phase 5 |
| Turbine/Generator | MANUAL | THIS PHASE |

# 2. TURBINE/GENERATOR PARAMETERS

| Parameter | Value | Notes |
| --- | --- | --- |
| Rated Output | ~1150 MW | Electrical |
| Rated Speed | 1800 rpm | 60 Hz, 4-pole |
| Steam Flow (100%) | 15.2×10⁶ lb/hr | From 4 SGs |
| Inlet Pressure | 1000 psia | From SGs |
| Efficiency | ~34% | 3411→1150 MW |
| Governor Droop | 5% | 5% Δspeed=100% load |
| Grid Frequency | 60.00 Hz | ±0.05 Hz sync |

## 2.1 Xenon Reactivity (ADDED)

Xenon-135 is the most significant fission product poison.

| Parameter | Value | Notes |
| --- | --- | --- |
| Equilibrium Xe worth | ~2500-3000 pcm | At 100% power |
| Peak Xe worth | ~3000-4000 pcm | After trip |
| Time to peak | 8-10 hours | After trip |
| Xe decay to 50% | ~24 hours | After peak |

Xenon buildup after trip can prevent restart for 8-24 hours ('xenon precluded').

## 2.2 Decay Heat (ADDED)

Decay heat continues after shutdown (ANS 5.1-2005 standard).

| Time After Trip | Decay Heat | Power (MW) |
| --- | --- | --- |
| Immediately | ~7% | ~239 MW |
| 1 minute | ~5% | ~171 MW |
| 10 minutes | ~3% | ~102 MW |
| 1 hour | ~1.5% | ~51 MW |
| 1 day | ~0.5% | ~17 MW |

Decay heat MUST be removed even after shutdown - drives emergency cooling requirements.

# 3. LOAD FOLLOWING DYNAMICS

Complete energy chain creates load-following challenge:

1. Operator increases load setpoint

2. Governor opens valves → more steam demand

3. Steam pressure drops → SG heat transfer increases

4. Tcold drops (more heat removed) → Tavg drops

5. Positive MTC → reactivity increases → power rises

6. New equilibrium at higher load

Xenon complicates this: Load change → Xe concentration changes → delayed reactivity effect

# 4. UNIT TEST SUITE (44 Tests)

## 4.1 Turbine Tests (20 tests)

| ID | Test | Expected | □ |
| --- | --- | --- | --- |
| TB-01 | Governor maintains 1800 rpm before sync | 1800 rpm | □ |
| TB-02 | Load control after sync | MW matches | □ |
| TB-03 | Droop: 5% speed drop = 100% load | 5% | □ |
| TB-04 | Frequency = speed/30 (4-pole) | 60 Hz | □ |
| TB-05 | Sync check: freq, volt, phase | All 3 | □ |
| TB-06 | Generator output = turbine × 0.985 | 98.5% | □ |

+ 14 additional turbine tests

## 4.2 Xenon Tests (12 tests)

| ID | Test | Expected | □ |
| --- | --- | --- | --- |
| XE-01 | Equilibrium Xe at 100% power | 2500-3000 pcm | □ |
| XE-02 | Xe peaks 8-10 hr after trip | 8-10 hr | □ |
| XE-03 | Peak Xe worth 3000-4000 pcm | 3000-4000 | □ |
| XE-04 | Xe decays with ~9 hr half-life | ~9 hr | □ |
| XE-05 | I-135 → Xe-135 decay modeled | Correct | □ |
| XE-06 | Xe burnout at high flux | Reduces | □ |

+ 6 additional xenon tests

## 4.3 Decay Heat Tests (12 tests)

| ID | Test | Expected | □ |
| --- | --- | --- | --- |
| DH-01 | Decay heat ~7% immediately after trip | ~7% | □ |
| DH-02 | Decay heat ~1.5% at 1 hour | ~1.5% | □ |
| DH-03 | Decay heat ~0.5% at 1 day | ~0.5% | □ |
| DH-04 | ANS 5.1-2005 curve followed | Matches | □ |
| DH-05 | Core heats up without cooling | Temp rises | □ |
| DH-06 | Decay heat requires active removal | Required | □ |

+ 6 additional decay heat tests

# 5. INTEGRATION TEST SUITE (24 Tests)

## 5.1 Full Plant Tests (12 tests)

| ID | Scenario | Criteria | □ |
| --- | --- | --- | --- |
| INT-601 | Complete startup: cold → 100% | No trips | □ |
| INT-602 | Load follow 100%→50%→100% with Xe | Manages Xe | □ |
| INT-603 | Generator trip (load rejection) | Plant stable | □ |
| INT-604 | Turbine trip: Rx trips, stabilizes | Safe state | □ |
| INT-605 | Grid sync successful | Sync OK | □ |
| INT-606 | Steady state: all params in tolerance | All pass | □ |

+ 6 additional full plant tests

## 5.2 Transient Tests (12 tests)

| ID | Scenario | Criteria | □ |
| --- | --- | --- | --- |
| INT-607 | Trip from 100%: decay heat visible | ~7% initial | □ |
| INT-608 | Post-trip: Xe buildup visible | Peak 8-10 hr | □ |
| INT-609 | Restart attempt during Xe peak | May be precluded | □ |
| INT-610 | Natural circulation after RCP loss | Decay heat removed | □ |
| INT-611 | Loss of feedwater with decay heat | SG level drops | □ |

+ 7 additional transient tests

# 6. FINAL ACCEPTANCE CRITERIA

| Parameter | Value @ 100% | Tolerance | Source |
| --- | --- | --- | --- |
| Reactor Power | 3411 MW | ±20 MW | FSAR |
| Generator Output | ~1150 MW | ±20 MW | FSAR |
| Tavg | 588.5°F | ±2°F | FSAR |
| RCS Pressure | 2235 psig | ±10 psig | FSAR |
| Steam Pressure | 1000 psia | ±20 psia | FSAR |
| Frequency | 60.00 Hz | ±0.02 Hz | Grid |

# 7. PHASE 6 EXIT GATE (PROJECT COMPLETE)

| Requirement | Status | Date |
| --- | --- | --- |
| Complete startup: cold → 100% | □ PASS | ________ |
| Load follow with xenon management | □ PASS | ________ |
| Decay heat modeled correctly | □ PASS | ________ |
| Reactor trip: plant stabilizes | □ PASS | ________ |
| All steady-state params in tolerance | □ PASS | ________ |
| All 44 unit tests pass | □ PASS | ________ |
| All 24 integration tests pass | □ PASS | ________ |
| Phase 1-5 tests still passing | □ PASS | ________ |
| PHASE 6 GATE APPROVED | □ YES | ________ |

# CRITICAL: MASTER THE ATOM - COMPLETE

✓ Validated physics engine with 13 gap fixes (Phase 1: 112 tests)

✓ Reactor core with kinetics and feedback (Phase 2: 85 tests)

✓ Pressurizer with non-equilibrium dynamics (Phase 3: 78 tests)

✓ CVCS with boron transport delay (Phase 4: 65 tests)

✓ RCPs with coastdown & natural circulation 12-23k gpm (Phase 4)

✓ Steam generators with shrink/swell (Phase 5: 72 tests)

✓ Turbine-generator with governor control (Phase 6: 68 tests)

✓ Xenon dynamics with load-following impact

✓ Decay heat curve (ANS 5.1-2005)

✓ Complete protection system

TOTAL: 480 validated tests across 6 phases

The simulator provides a realistic, challenging PWR operations experience.