PHASE 1

CORE PHYSICS ENGINE

Implementation Plan

"Get the physics right"

Duration: 2-3 weeks

# 1. Phase Overview

Phase 1 establishes the foundational physics that all other systems depend on. This is standalone code with no UI - just pure physics calculations that can be validated against known values before any simulator infrastructure is built.

The deliverable is a physics library that answers questions like: What is the saturation temperature at 2250 psia? How much does 11,500 ft³ of water expand when heated from 200°F to 580°F? What pressure results from compressing a 720 ft³ steam bubble to 600 ft³?

## 1.1 Why Physics First?

Phase 0 failed because we embedded incorrect physics constants into complex code, making debugging nearly impossible. By isolating physics into a validated library first, we ensure every downstream calculation starts from correct foundations.

This phase produces no visible simulator - just validated functions. But it's the most critical phase, because errors here propagate everywhere.

## 1.2 Success Criteria

The physics library passes a comprehensive test suite that validates each function against published steam tables (IAPWS-IF97), textbook thermodynamics, and known plant data. No function is accepted until its test passes.

# 2. Module 1: Water/Steam Properties

This module provides thermodynamic properties of water and steam as functions of temperature and pressure.

## 2.1 Required Functions

### 2.1.1 Saturation Pressure

Given temperature, return saturation pressure. Critical for subcooling margin calculations.

function saturationPressure(T_fahrenheit) → pressure_psia

Implementation: Use Antoine equation for T < 212°F, Wagner equation (IAPWS-IF97) for T ≥ 212°F. The critical point is 705.4°F at 3206.2 psia.

### 2.1.2 Saturation Temperature

Given pressure, return saturation temperature. Critical for pressurizer steam space.

function saturationTemperature(P_psia) → temperature_F

Implementation: Inverse of saturation pressure. At 2250 psia, Tsat = 652.9°F.

### 2.1.3 Water Density

Given temperature and pressure, return water density. Critical for mass/volume calculations.

function waterDensity(T_fahrenheit, P_psia) → density_lb_per_ft3

Key values: 62.4 lb/ft³ at 60°F, ~45 lb/ft³ at 580°F/2250 psia, ~37 lb/ft³ near saturation.

### 2.1.4 Water Specific Heat

Given temperature and pressure, return specific heat capacity.

function waterSpecificHeat(T_fahrenheit, P_psia) → Cp_BTU_per_lb_F

Key values: 1.0 BTU/lb·°F at 60°F, ~1.2 BTU/lb·°F at operating conditions, rises sharply near saturation.

### 2.1.5 Thermal Expansion Coefficient

Given temperature and pressure, return volumetric thermal expansion coefficient.

function thermalExpansionCoeff(T_fahrenheit, P_psia) → beta_per_F

Key values: ~0.0001/°F at low T, ~0.0025/°F at operating conditions.

## 2.2 Validation Targets

| Function | Input | Expected Output | Tolerance |
| --- | --- | --- | --- |
| saturationPressure | 212°F | 14.7 psia | ±0.1 psia |
| saturationPressure | 653°F | 2250 psia | ±5 psia |
| saturationTemperature | 2250 psia | 652.9°F | ±1°F |
| waterDensity | 580°F, 2250 psia | ~45 lb/ft³ | ±2 lb/ft³ |
| waterSpecificHeat | 580°F, 2250 psia | ~1.2 BTU/lb·°F | ±0.1 |

# 3. Module 2: Steam Space Thermodynamics

This module handles the pressurizer steam bubble behavior - compression, expansion, and condensation.

## 3.1 Required Functions

### 3.1.1 Polytropic Compression

Calculate pressure change from volume change in the steam space.

function polytropicPressure(P1, V1, V2, n=1.3) → P2

Physics: P₁V₁ⁿ = P₂V₂ⁿ where n=1.3 for steam (between isothermal n=1 and adiabatic n=1.33).

Example: Compressing 720 ft³ to 600 ft³ at 2250 psia → ~2850 psia (before spray/relief).

### 3.1.2 Steam Compressibility

Return the fractional volume change per psi for the steam space.

function steamCompressibility(P_psia, T_fahrenheit) → dV_V_per_psi

At operating conditions: ~2.2×10⁻⁴ per psi (NOT 3×10⁻⁶ as in Phase 0).

### 3.1.3 Flash/Condensation Rate

Calculate mass transfer between water and steam phases based on temperature difference from saturation.

function phaseChangeRate(T_water, T_sat, surfaceArea, heatTransferCoeff) → mass_rate_lb_per_sec

When water is above Tsat, it flashes to steam. When steam contacts subcooled water (spray), it condenses.

## 3.2 Validation Targets

| Scenario | Expected Result | Tolerance |
| --- | --- | --- |
| 720 ft³ → 600 ft³ compression | 2250 → ~2850 psia | ±50 psia |
| 720 ft³ → 800 ft³ expansion | 2250 → ~1900 psia | ±50 psia |
| Steam compressibility at 2250 psia | ~2.2×10⁻⁴ /psi | ±20% |

# 4. Module 3: Thermal Mass & Heat Transfer

This module calculates heat storage in water and metal, plus heat transfer between components.

## 4.1 Required Functions

### 4.1.1 Water Thermal Mass

Calculate heat capacity of a water volume at given conditions.

function waterThermalMass(volume_ft3, T_fahrenheit, P_psia) → BTU_per_F

Formula: mass × Cp = (volume × density) × specific heat

Example: 11,500 ft³ at 580°F/2250 psia → 11,500 × 45 × 1.2 ≈ 620,000 BTU/°F

### 4.1.2 Metal Thermal Mass

Calculate heat capacity of steel components.

function metalThermalMass(mass_lb) → BTU_per_F

Formula: mass × Cp = mass × 0.12 BTU/lb·°F for steel

Example: 2,200,000 lb steel → 264,000 BTU/°F

### 4.1.3 Combined System Thermal Mass

Total thermal mass of RCS water + metal structures.

function systemThermalMass(waterVolume, metalMass, T, P) → BTU_per_F

At operating conditions: ~620,000 (water) + 264,000 (metal) ≈ 884,000 BTU/°F

This means adding 884 MW-seconds of heat raises the system by 1°F.

### 4.1.4 Heat Transfer Rate

Calculate heat flow between two temperature nodes.

function heatTransferRate(T_hot, T_cold, UA_coefficient) → BTU_per_sec

Formula: Q = UA × ΔT where UA is the overall heat transfer coefficient × area.

## 4.2 Validation Targets

| Calculation | Expected Result | Tolerance |
| --- | --- | --- |
| RCS water thermal mass at operating T | ~620,000 BTU/°F | ±10% |
| Metal thermal mass (2.2M lb) | ~264,000 BTU/°F | ±10% |
| 3411 MW into system for 1 second | ΔT ≈ 3.7°F | ±0.5°F |

# 5. Module 4: Thermal Expansion

This module calculates volume changes as the RCS heats and cools - critical for pressurizer response.

## 5.1 Required Functions

### 5.1.1 Water Volume at Temperature

Given a reference mass of water, calculate its volume at different temperatures.

function waterVolumeAtTemp(mass_lb, T_fahrenheit, P_psia) → volume_ft3

Formula: volume = mass / density(T, P)

### 5.1.2 Expansion from Temperature Change

Calculate volume change for a given temperature change.

function thermalExpansion(volume_ft3, T_initial, T_final, P_psia) → delta_volume_ft3

Example: 11,500 ft³ heated from 200°F to 580°F → expands by ~1,240 ft³ (10.8%)

### 5.1.3 Surge Rate

Calculate the rate of volume flow into/out of pressurizer from temperature change rate.

function surgeRate(rcsVolume, dT_dt, T, P) → flow_ft3_per_sec

Formula: surge = V × β × dT/dt where β is thermal expansion coefficient

## 5.2 Validation Targets

| Scenario | Expected Result | Tolerance |
| --- | --- | --- |
| Cold (200°F) to hot (580°F) expansion | ~1,240 ft³ (10.8% of RCS) | ±100 ft³ |
| 1°F/min heatup surge rate | ~0.5 ft³/sec into pressurizer | ±0.1 ft³/sec |
| Full power ΔT (61°F) expansion | ~150 ft³ | ±20 ft³ |

# 6. Module 5: Reactor Physics

This module implements point kinetics and reactivity feedback - the nuclear-specific physics.

## 6.1 Required Functions

### 6.1.1 Point Kinetics Solver

Solve the coupled differential equations for neutron population and delayed precursors.

function pointKinetics(n, precursors[], rho, dt) → {n_new, precursors_new[]}

Uses 6-group delayed neutron model with β = 0.0065, Λ = 20 μs.

Implementation: Use prompt jump approximation for stability at large timesteps.

### 6.1.2 Doppler Feedback

Calculate reactivity change from fuel temperature.

function dopplerReactivity(T_fuel, T_fuel_ref) → rho_pcm

Formula: Δρ = α_D × (√T_fuel - √T_ref) where α_D ≈ -2.5 pcm/√°F

Typical Doppler defect (cold to hot): ~1,200 pcm

### 6.1.3 Moderator Temperature Feedback

Calculate reactivity change from moderator (coolant) temperature.

function moderatorReactivity(T_mod, T_mod_ref, boronPPM) → rho_pcm

MTC varies with boron: +5 pcm/°F at high boron (BOL), -40 pcm/°F at low boron (EOL).

### 6.1.4 Boron Reactivity

Calculate reactivity worth of dissolved boron.

function boronReactivity(boronPPM, boronPPM_ref) → rho_pcm

Worth varies: ~10 pcm/ppm at BOL, ~8 pcm/ppm at EOL.

### 6.1.5 Control Rod Worth

Calculate reactivity from rod position.

function rodReactivity(rodPosition_steps, rodWorthCurve) → rho_pcm

Uses integral rod worth curve. Total worth ~1,000-1,500 pcm for control bank.

## 6.2 Validation Targets

| Scenario | Expected Result | Tolerance |
| --- | --- | --- |
| Step reactivity +50 pcm | Power rises ~8%/sec initially | ±2%/sec |
| Doppler defect (0→100% power) | ~1,200 pcm | ±200 pcm |
| MTC at EOL, low boron | -30 to -50 pcm/°F | within range |
| 100 ppm boron change | ~900 pcm reactivity | ±100 pcm |

# 7. Module 6: Fluid Flow

This module calculates flow rates and pressure drops through the RCS.

## 7.1 Required Functions

### 7.1.1 Pump Head/Flow Curve

Given pump speed, calculate head and flow.

function pumpCurve(speed_rpm, systemResistance) → {head_ft, flow_gpm}

RCP nominal: 97,600 gpm at 280 ft head, 1189 rpm, ~7,000 HP.

### 7.1.2 System Resistance

Calculate pressure drop as function of flow rate.

function systemPressureDrop(flow_gpm) → deltaP_psi

ΔP ∝ Q² (turbulent flow). Size to give 280 ft head at nominal flow.

### 7.1.3 Natural Circulation Flow

Calculate flow rate driven by density difference (no pumps).

function naturalCirculation(T_hot, T_cold, loopHeight_ft) → flow_gpm

Driving head = Δρ × g × h. Typical NC flow ~5% of forced flow.

## 7.2 Validation Targets

| Scenario | Expected Result | Tolerance |
| --- | --- | --- |
| 4 RCPs at full speed | 390,400 gpm total | ±5% |
| 1 RCP trip (3 running) | ~75% flow | ±5% |
| Natural circulation (61°F ΔT) | ~5% nominal flow | ±2% |

# 8. Test Framework

Every function must pass its test suite before integration. Tests use known values from steam tables and plant data.

## 8.1 Test Structure

describe('WaterProperties', () => {

test('saturation pressure at 212F equals 14.7 psia', () => {

expect(saturationPressure(212)).toBeCloseTo(14.7, 1);

});

test('saturation pressure at 653F equals 2250 psia', () => {

expect(saturationPressure(653)).toBeCloseTo(2250, 0);

});

});

## 8.2 Reference Data Sources

NIST/IAPWS-IF97 Steam Tables - Authoritative source for water/steam properties

Westinghouse PWR Manual - Plant-specific data and operating conditions

NRC Technical Training - Typical coefficient values and transient responses

# 9. Deliverables

## 9.1 Code Artifacts

physics/water-properties.js - Water and steam thermodynamic functions

physics/steam-thermodynamics.js - Steam space compression/expansion

physics/thermal-mass.js - Heat capacity calculations

physics/thermal-expansion.js - Volume change calculations

physics/reactor-kinetics.js - Point kinetics and reactivity feedback

physics/fluid-flow.js - Pump curves and pressure drops

physics/index.js - Unified export of all physics functions

## 9.2 Test Artifacts

tests/water-properties.test.js - Steam table validation

tests/steam-thermodynamics.test.js - Compression validation

tests/thermal-expansion.test.js - Expansion validation

tests/reactor-kinetics.test.js - Kinetics validation

tests/integration.test.js - Cross-module validation

## 9.3 Documentation

PHYSICS.md - Function reference with equations and assumptions

VALIDATION.md - Test results and comparison to reference data

## 9.4 Exit Criteria

All unit tests pass with specified tolerances

Integration test: Full heatup from 200°F to 580°F produces ~1,240 ft³ expansion

Integration test: 10% power step produces realistic Tavg transient (peak ~5°F, settling ~60s)

Code review confirms equations match documented physics

Phase 1 Complete → Proceed to Phase 2 (Reactor Core)