PHASE 2

REACTOR CORE

with Automated Auxiliaries

Implementation Plan

"Build the reactor, automate everything else"

Duration: 3-4 weeks

# 1. Phase Overview

Phase 2 builds the reactor core simulation with all supporting systems running in automatic mode. The operator can only interact with the reactor - control rods and power - while everything else (pressurizer, SGs, CVCS, turbine) runs perfectly in the background.

This approach lets us validate the core reactor physics and thermal-hydraulics before adding complexity. If the reactor behaves correctly with perfect boundary conditions, we know the fundamental model is right.

## 1.1 Architecture Concept

The simulation has two types of systems:

MANUAL (Phase 2): Reactor core, control rods - operator controlled

AUTO (Phase 2): Everything else - perfect boundary conditions

Auto systems are 'magic boxes' that maintain setpoints perfectly. The pressurizer holds 2235 psig. The SGs hold steam pressure. The turbine follows load demand. No dynamics, no lag, just perfect control.

## 1.2 Why This Approach?

Complex systems fail in complex ways. By automating auxiliaries, any problems we see MUST come from the reactor model. This makes debugging tractable. Once the reactor works, we progressively replace auto systems with manual ones.

## 1.3 Success Criteria

Reactor can be taken critical, raised to power, and responds correctly to rod motion and temperature changes. Automatic systems maintain stable boundary conditions without operator intervention.

# 2. System Architecture

## 2.1 State Variables

The simulation maintains a central state object updated each timestep:

state = {

// REACTOR (manual)

neutronPower: 0.0,        // 0-1 fraction of 3411 MW

thermalPower: 0.0,        // MW, lags neutron power

fuelTemp: 547,            // °F average fuel temperature

reactivity: 0,            // pcm total

rodPosition: 0,           // 0-228 steps

boronConcentration: 1500, // ppm

// RCS (computed)

Tcold: 547,               // °F cold leg

Thot: 547,                // °F hot leg

Tavg: 547,                // °F average

pressure: 2235,           // psig (held by auto pressurizer)

flow: 390400,             // gpm (held by auto RCPs)

// SECONDARY (auto)

steamPressure: 1000,      // psia (held by auto SGs)

steamFlow: 0,             // lb/hr (follows power)

feedwaterFlow: 0,         // lb/hr (matches steam)

// TURBINE (auto)

turbinePower: 0,          // MW electrical

loadDemand: 0,            // MW setpoint

}

## 2.2 Simulation Loop

function simulationStep(dt) {

// 1. Calculate reactivity from all sources

const rho = calculateReactivity(state);

// 2. Solve point kinetics for neutron power

state.neutronPower = solveKinetics(state.neutronPower, rho, dt);

// 3. Calculate thermal power (lagged)

state.thermalPower = thermalLag(state.neutronPower, state.thermalPower, dt);

// 4. Calculate temperatures from heat balance

updateTemperatures(state, dt);

// 5. Auto systems maintain their setpoints (no dynamics)

autoSystems.update(state);

}

# 3. Reactor Core Model

## 3.1 Neutron Power

Neutron power comes from point kinetics using Phase 1 physics:

function solveKinetics(power, reactivity, dt) {

// Use prompt jump approximation for stability

// dn/dt = (rho - beta)/Lambda * n + sum(lambda_i * c_i)

// Stable for dt up to ~100ms

}

## 3.2 Thermal Power

Thermal power lags neutron power due to fuel heat capacity:

function thermalLag(neutronPower, thermalPower, dt) {

const tau = 5.0; // seconds, fuel time constant

return thermalPower + (neutronPower - thermalPower) * (1 - Math.exp(-dt/tau));

}

## 3.3 Fuel Temperature

Fuel temperature rises with power, providing Doppler feedback:

// Simplified: T_fuel = T_coolant + k * Power

// k sized to give ~1200 pcm Doppler defect at full power

state.fuelTemp = state.Tavg + 400 * state.thermalPower; // ~950°F at full power

## 3.4 Reactivity Calculation

Total reactivity is the sum of all contributions:

function calculateReactivity(state) {

let rho = 0;

// Control rod worth (integral curve)

rho += rodWorth(state.rodPosition);

// Doppler (fuel temperature)

rho += dopplerReactivity(state.fuelTemp, T_REF);

// Moderator temperature

rho += moderatorReactivity(state.Tavg, T_REF, state.boronConcentration);

// Boron

rho += boronReactivity(state.boronConcentration, BORON_REF);

return rho;

}

# 4. RCS Temperature Model

## 4.1 Heat Balance

The RCS is a closed loop: heat added in core, removed in steam generators.

Q_core = thermalPower * 3411e6 * 3.412; // BTU/hr

Q_SG = steamFlow * (h_steam - h_feedwater);  // BTU/hr

Q_net = Q_core - Q_SG; // positive = heating

## 4.2 Temperature Rise Across Core

Core ΔT depends on power and flow:

// ΔT = Q / (m_dot * Cp)

// At 100% power, 100% flow: ΔT = 61°F

state.deltaT = 61 * (state.thermalPower / state.flow) * (NOMINAL_FLOW);

state.Thot = state.Tcold + state.deltaT;

state.Tavg = (state.Thot + state.Tcold) / 2;

## 4.3 Simplified Single-Node Model

For Phase 2, use single-node RCS (Tavg tracks power):

function updateTemperatures(state, dt) {

// Heat added by core

const Q_core = state.thermalPower * 3411e6 * 3.412; // BTU/hr

// Heat removed by SGs (auto-matched to maintain Tavg program)

const Tavg_target = tavgProgram(state.thermalPower);

const Q_SG = Q_core + (state.Tavg - Tavg_target) * THERMAL_MASS / dt;

// Net heat into RCS

const Q_net = Q_core - Q_SG;

// Temperature change

state.Tavg += Q_net * dt / (THERMAL_MASS * 3600);

// Calculate Thot and Tcold from Tavg and ΔT

state.deltaT = 61 * state.thermalPower;

state.Thot = state.Tavg + state.deltaT / 2;

state.Tcold = state.Tavg - state.deltaT / 2;

}

## 4.4 Tavg Program

Tavg varies linearly with power to optimize efficiency:

function tavgProgram(power) {

// 547°F at 0% power, 588°F at 100% power

return 547 + 41 * power;

}

# 5. Automatic Systems (Boundary Conditions)

These systems maintain setpoints perfectly. No operator control, no dynamics. They are 'magic' for Phase 2.

## 5.1 Auto Pressurizer

class AutoPressurizer {

update(state) {

state.pressure = 2235; // psig, always

state.przLevel = przLevelProgram(state.Tavg); // 25% at HZP, 60% at HFP

}

}

Real pressurizer dynamics come in Phase 3.

## 5.2 Auto Steam Generators

class AutoSteamGenerators {

update(state) {

state.steamPressure = 1000; // psia, always

state.steamFlow = state.thermalPower * FULL_STEAM_FLOW;

state.feedwaterFlow = state.steamFlow; // perfect match

state.sgLevel = 50; // percent, always

}

}

Real SG dynamics come in Phase 5.

## 5.3 Auto RCPs

class AutoRCPs {

update(state) {

state.flow = 390400; // gpm, all 4 pumps running

}

}

RCP trips and coastdown come in Phase 4.

## 5.4 Auto CVCS

class AutoCVCS {

update(state) {

// Maintains RCS inventory and boron as commanded

// No delay, instant response

state.boronConcentration = this.boronSetpoint;

}

setBoron(ppm) {

this.boronSetpoint = ppm; // instant, no delay

}

}

Real CVCS with delays comes in Phase 4.

## 5.5 Auto Turbine/Generator

class AutoTurbine {

update(state) {

// Perfect conversion, instant response

state.turbinePower = state.thermalPower * 1150 / 3411; // ~34% efficiency

state.generatorOutput = state.turbinePower;

}

}

Real turbine control comes in Phase 6.

# 6. Control Rod System (Manual)

The only manual control in Phase 2 is rod position.

## 6.1 Rod Motion

const ROD_SPEED = 72; // steps per minute

const ROD_MAX = 228;  // steps, fully withdrawn

const ROD_MIN = 0;    // steps, fully inserted

function moveRods(direction, dt) {

const movement = (direction * ROD_SPEED / 60) * dt;

state.rodPosition = clamp(state.rodPosition + movement, ROD_MIN, ROD_MAX);

}

## 6.2 Rod Worth Curve

Integral rod worth follows typical S-curve:

function rodWorth(position) {

// Normalized position 0-1

const x = position / ROD_MAX;

// S-curve: low worth at top/bottom, high worth in middle

// Total worth ~1000 pcm

return 1000 * (3*x*x - 2*x*x*x);

}

## 6.3 Operator Interface

Simple controls for Phase 2:

// Keyboard/button controls

'R' or UP_ARROW    → withdraw rods

'F' or DOWN_ARROW  → insert rods

RELEASE            → stop rod motion

// Emergency

SPACEBAR           → reactor trip (all rods to bottom)

# 7. User Interface (Minimal)

Phase 2 UI shows key parameters. No control panels for auto systems.

## 7.1 Display Parameters

| Parameter | Units | Typical Range |
| --- | --- | --- |
| Neutron Power | % | 0-100 (can exceed in transient) |
| Thermal Power | MW | 0-3411 |
| Reactivity | pcm | -5000 to +500 |
| Rod Position | steps | 0-228 |
| Tavg | °F | 547-588 |
| Thot / Tcold | °F | 547-619 / 547-558 |
| Pressure | psig | 2235 (constant in Phase 2) |
| Boron | ppm | 0-2000 |
| Startup Rate | DPM | -1 to +1 (decades per minute) |

## 7.2 Status Indicators

Reactor Status: SHUTDOWN / CRITICAL / POWER OPERATION

Control Rods: INSERTING / STOPPED / WITHDRAWING / TRIP

All Auto Systems: [AUTO] displayed, no manual control available

## 7.3 Trend Displays

Rolling 5-minute history of: Power, Tavg, Reactivity

Essential for observing dynamics during startup and transients.

# 8. Test Scenarios

## 8.1 Reactor Startup

Starting condition: Cold shutdown, rods at 0, boron at 2000 ppm

Procedure:

1. Reduce boron to ~1500 ppm (auto CVCS, instant in Phase 2)

2. Withdraw rods slowly, watching startup rate

3. Achieve criticality (reactivity = 0, SUR = 0)

4. Continue withdrawal to establish positive SUR (~0.5 DPM)

5. Power rises exponentially until heat adds negative reactivity

6. Establish 'point of adding heat' at ~1% power

7. Continue power ascension with rod withdrawal to compensate feedback

## 8.2 Power Maneuver

Starting condition: 50% power, equilibrium

Test: Withdraw rods to increase to 75% power

Expected:

- Initial positive reactivity from rod withdrawal

- Power rises, fuel heats up, Doppler adds negative reactivity

- Tavg rises, moderator adds negative reactivity

- Power stabilizes when negative feedback balances rod worth

- Final equilibrium at higher power with higher Tavg

## 8.3 Reactor Trip

Starting condition: 100% power

Test: Press trip button

Expected:

- All rods insert rapidly (~2 seconds to bottom)

- Reactivity goes very negative (~-5000 pcm)

- Neutron power drops to <5% in ~3 seconds

- Thermal power decays on fuel time constant

- Tavg drops as SGs continue removing heat

# 9. Validation Criteria

## 9.1 Steady State

| Condition | Expected Value | Tolerance |
| --- | --- | --- |
| 100% power Tavg | 588°F | ±2°F |
| 100% power ΔT | 61°F | ±3°F |
| 100% power Thot | 619°F | ±3°F |
| Equilibrium reactivity | 0 pcm | ±5 pcm |

## 9.2 Transient Response

| Transient | Expected Behavior | Criterion |
| --- | --- | --- |
| +100 pcm step (rod withdrawal) | Power rises, stabilizes higher | No oscillation, settling <60s |
| Reactor trip from 100% | Power <5% in 3 seconds | Smooth decay, no rebound |
| 10% power increase via rods | Tavg rises ~4°F | Follows Tavg program |

# 10. Deliverables

## 10.1 Code Artifacts

core/reactor.js - Reactor core model (kinetics, temperatures)

core/control-rods.js - Rod motion and worth curves

auto/pressurizer.js - Auto pressurizer boundary

auto/steam-generators.js - Auto SG boundary

auto/cvcs.js - Auto CVCS boundary

auto/rcps.js - Auto RCP boundary

auto/turbine.js - Auto turbine boundary

simulation/loop.js - Main simulation timestep

simulation/state.js - State management

## 10.2 UI Artifacts

ui/display.js - Parameter display

ui/controls.js - Rod control interface

ui/trends.js - Rolling trend displays

## 10.3 Test Artifacts

tests/reactor.test.js - Reactor model unit tests

tests/scenarios.test.js - Full scenario validation

## 10.4 Exit Criteria

Reactor achieves criticality through rod withdrawal

Power responds correctly to rod motion (negative feedback works)

Reactor trip rapidly reduces power

Steady-state temperatures match Tavg program

All auto systems maintain setpoints without drift

Phase 2 Complete → Proceed to Phase 3 (Pressurizer)