PHASE 3

PRESSURIZER SYSTEM

Auto → Manual Conversion

Implementation Plan

"First system swap: pressure control with real dynamics"

Duration: 2-3 weeks

# 1. Phase Overview

Phase 3 replaces the auto pressurizer with a physics-based model. The pressurizer becomes the first system that can 'fight back' - it responds to thermal expansion with real dynamics, and the operator must manage pressure and level.

This is the critical test of our physics library. The pressurizer integrates steam compressibility, thermal expansion, and heat transfer. If our Phase 1 constants are correct, the pressurizer will respond realistically.

## 1.1 System Status After Phase 3

| System | Status |
| --- | --- |
| Reactor Core | MANUAL (from Phase 2) |
| Pressurizer | MANUAL (this phase) |
| Steam Generators | AUTO |
| CVCS | AUTO |
| RCPs | AUTO |
| Turbine/Generator | AUTO |

## 1.2 Success Criteria

Pressure responds to temperature transients with correct magnitude and timing. Heaters and spray can control pressure within normal operating band. Level changes with thermal expansion.

# 2. Pressurizer Physics

## 2.1 Two-Region Model

The pressurizer contains two regions: liquid (bottom) and steam (top). The interface between them is at saturation conditions.

state.pressurizer = {

pressure: 2235,      // psig

liquidVolume: 1080,  // ft³ (60% of 1800 at normal)

steamVolume: 720,    // ft³ (40% at normal)

liquidTemp: 653,     // °F (saturation at 2235 psig)

steamTemp: 653,      // °F (saturation)

level: 60,           // percent

heaterPower: 0,      // kW (0-1800)

sprayFlow: 0,        // gpm (0-900)

}

## 2.2 Pressure Dynamics

Pressure changes come from three sources:

1. Volume change (surge): Water entering/leaving compresses/expands steam

2. Heat addition (heaters): Boils water, increasing steam volume

3. Cooling (spray): Condenses steam, reducing steam volume

### 2.2.1 Surge Effect

// Water surging IN compresses steam → pressure rises

// P2 = P1 * (V1/V2)^n where n = 1.3

function surgePressure(P1, steamVol1, surgeVol) {

const steamVol2 = steamVol1 - surgeVol; // steam compressed

return P1 * Math.pow(steamVol1/steamVol2, 1.3);

}

### 2.2.2 Heater Effect

// Heaters boil water at interface → steam mass increases

// At saturation, adding heat creates steam

function heaterSteamGeneration(heaterPower_kW, pressure) {

const hfg = latentHeat(pressure); // BTU/lb

const Q = heaterPower_kW * 3412 / 3600; // BTU/sec

return Q / hfg; // lb/sec of steam created

}

### 2.2.3 Spray Effect

// Spray condenses steam → steam mass decreases

// Cold water (~550°F) contacts saturated steam (653°F)

function sprayCondensation(sprayFlow_gpm, T_spray, T_sat) {

const massFlow = sprayFlow_gpm * 8.34 / 60; // lb/sec

const Q = massFlow * Cp * (T_sat - T_spray); // BTU/sec absorbed

const hfg = latentHeat(2235); // BTU/lb

return Q / hfg; // lb/sec of steam condensed

}

# 3. Level Dynamics

## 3.1 Level Calculation

Level is simply liquid volume divided by total volume:

state.pressurizer.level = (liquidVolume / 1800) * 100; // percent

## 3.2 Surge Flow

Water flows between RCS and pressurizer through the surge line:

function calculateSurgeFlow(state, dt) {

// Thermal expansion drives surge

const rcsExpansion = thermalExpansion(

state.rcsVolume, state.Tavg_old, state.Tavg, state.pressure

);

// Positive = water entering pressurizer (outsurge from RCS heating)

// Negative = water leaving pressurizer (insurge from RCS cooling)

return rcsExpansion / dt; // ft³/sec

}

## 3.3 Level Program

Desired level varies with Tavg to accommodate thermal expansion:

function levelProgram(Tavg) {

// ~25% at 547°F (HZP), ~60% at 588°F (HFP)

const fraction = (Tavg - 547) / (588 - 547);

return 25 + 35 * fraction;

}

## 3.4 Level Limits

| Level | Condition | Action | Consequence |
| --- | --- | --- | --- |
| <15% | LOW-LOW | Reactor trip | Heaters uncover risk |
| 15-20% | LOW | Alarm, max charging | Restore inventory |
| 20-80% | NORMAL | Auto level control | Normal operation |
| 80-85% | HIGH | Alarm, max letdown | Reduce inventory |
| >92% | HIGH-HIGH | Trip heaters | Solid pressurizer risk |

# 4. Pressure Control

## 4.1 Control Setpoints

| Setpoint | Pressure | Action | Function |
| --- | --- | --- | --- |
| Spray Full On | >2280 psig | 900 gpm spray | Rapid pressure reduction |
| Spray On | >2260 psig | Proportional spray | Moderate overpressure |
| Normal | 2235 psig | Proportional heaters | Maintain setpoint |
| Heaters Full On | <2210 psig | 1800 kW heaters | Restore pressure |
| PORV Open | >2335 psig | Relief valve lifts | Prevent safety lift |
| Safety Valve | >2485 psig | Code safety opens | Ultimate protection |

## 4.2 Controller Implementation

function pressureController(state, dt) {

const error = state.pressurizer.pressure - 2235;

// Spray control (high pressure)

if (state.pressurizer.pressure > 2280) {

state.pressurizer.sprayFlow = 900; // full spray

} else if (state.pressurizer.pressure > 2260) {

// Proportional spray 0-900 gpm over 2260-2280 range

state.pressurizer.sprayFlow = (error - 25) / 20 * 900;

} else {

state.pressurizer.sprayFlow = 0;

}

// Heater control (low pressure)

if (state.pressurizer.pressure < 2210) {

state.pressurizer.heaterPower = 1800; // all heaters

} else if (state.pressurizer.pressure < 2235) {

// Proportional heaters 0-1800 kW over 2210-2235 range

state.pressurizer.heaterPower = (25 - error) / 25 * 1800;

} else {

state.pressurizer.heaterPower = 0;

}

}

# 5. Integration with RCS

## 5.1 Coupling Points

The pressurizer connects to the RCS through:

1. Surge line (hot leg) - liquid flows in/out based on thermal expansion

2. Spray line (cold leg) - subcooled water for pressure control

3. Pressure equalization - RCS pressure equals pressurizer pressure

## 5.2 Updated Simulation Loop

function simulationStep(dt) {

// Reactor physics (from Phase 2)

updateReactor(state, dt);

updateTemperatures(state, dt);

// NEW: Pressurizer dynamics

const surgeFlow = calculateSurgeFlow(state, dt);

updatePressurizerVolumes(state, surgeFlow, dt);

updatePressurizerPressure(state, dt);

pressureController(state, dt);

// Propagate pressure to RCS

state.pressure = state.pressurizer.pressure;

// Other systems still auto

autoSteamGenerators.update(state);

autoCVCS.update(state);

autoTurbine.update(state);

}

## 5.3 Pressure Feedback to Reactor

RCS pressure affects reactor physics through saturation margin:

// Subcooling margin = Tsat(P) - T_actual

const subcooling = saturationTemperature(state.pressure) - state.Thot;

// Warning if subcooling < 30°F

// Trip if subcooling < 0 (boiling in core)

# 6. Operator Interface

## 6.1 New Displays

Pressurizer Panel:

- Pressure (psig) with trend

- Level (%) with trend

- Heater power (kW) with ON/OFF indicator per bank

- Spray flow (gpm)

- PORV status (CLOSED/OPEN)

- Relief tank level/temperature (if PORVs have lifted)

## 6.2 New Controls

Automatic mode (default):

- Pressure controller manages heaters and spray

- Level controller adjusts charging/letdown (via auto CVCS)

Manual mode (optional):

- Heater bank switches (ON/OFF for each bank)

- Spray valve switch (CLOSE/AUTO/OPEN)

- PORV block valve (OPEN/CLOSE)

## 6.3 Alarms

| Alarm | Setpoint | Priority |
| --- | --- | --- |
| PZR PRESS HIGH | >2260 psig | Caution |
| PZR PRESS LOW | <2210 psig | Caution |
| PZR PRESS HI-HI | >2335 psig | Warning (PORV) |
| PZR LEVEL HIGH | >80% | Caution |
| PZR LEVEL LOW | <20% | Caution |
| PZR LEVEL LO-LO | <15% | Trip |

# 7. Test Scenarios

## 7.1 Power Increase Transient

Starting: 50% power, 2235 psig, 45% level

Action: Raise power to 75% via rod withdrawal

Expected response:

- Tavg rises → RCS expands → water surges into pressurizer

- Surge compresses steam → pressure rises

- Spray activates at 2260 psig

- Level rises due to expansion

- Equilibrium at higher level, nominal pressure

## 7.2 Power Decrease Transient

Starting: 100% power, 2235 psig, 60% level

Action: Insert rods to reduce to 75% power

Expected response:

- Tavg drops → RCS contracts → water leaves pressurizer

- Steam expands → pressure drops

- Heaters activate at 2210 psig

- Level drops with contraction

- Equilibrium at lower level, nominal pressure

## 7.3 Loss of Heaters

Starting: 100% power, 2235 psig, normal heaters

Action: Trip all pressurizer heaters

Expected response:

- Steam condenses slowly on subcooled walls

- Pressure slowly decays

- If unaddressed, pressure drops to PORV closure point

- Operator must restore heaters or reduce letdown

## 7.4 Stuck Open PORV

Starting: 100% power, normal conditions

Action: PORV fails open

Expected response:

- Steam vents to relief tank → rapid pressure drop

- Heaters go full on (ineffective against steam loss)

- Level drops as steam mass lost

- Operator must close block valve

- Without action, potential for core uncovery

# 8. Validation Criteria

## 8.1 Steady State

| Parameter | Expected | Tolerance |
| --- | --- | --- |
| Pressure at steady state | 2235 psig | ±5 psig |
| Level at 100% power | ~60% | ±3% |
| Level at HZP | ~25% | ±3% |
| Heater power at equilibrium | ~300 kW (heat losses) | ±100 kW |

## 8.2 Transient Response

| Transient | Expected Response | Criterion |
| --- | --- | --- |
| 10% power increase | Pressure spike ~20 psi | Returns to 2235±10 in <60s |
| 10% power decrease | Pressure drop ~20 psi | Returns to 2235±10 in <60s |
| 100% load rejection | Pressure spike to ~2300 | No PORV lift (<2335) |
| Reactor trip | Pressure initially spikes, then drops | Level drops, heaters recover |

# 9. Deliverables

## 9.1 Code Artifacts

systems/pressurizer.js - Full pressurizer model

systems/pressurizer-controller.js - Pressure and level control

systems/relief-valves.js - PORV and safety valve logic

ui/pressurizer-panel.js - Pressurizer displays and controls

## 9.2 Test Artifacts

tests/pressurizer.test.js - Unit tests for pressurizer physics

tests/pressure-transients.test.js - Transient response validation

## 9.3 Exit Criteria

Pressure responds correctly to power transients (±20 psi for 10% step)

Heaters and spray maintain pressure within control band

Level tracks programmed value through power changes

PORV opens at setpoint, prevents safety valve lift

Steam bubble maintained (no solid pressurizer)

Phase 3 Complete → Proceed to Phase 4 (CVCS & RCPs)