PHASE 4

CVCS & RCP SYSTEMS

Inventory Control & Coolant Flow

"Boron control, charging/letdown, pump dynamics"

Duration: 2-3 weeks

# 1. Phase Overview

Phase 4 adds two critical systems: CVCS for inventory and boron control, and RCPs with real dynamics. These systems couple directly to the pressurizer (level/pressure) and reactor (boron reactivity, flow).

## 1.1 System Status After Phase 4

| System | Status |
| --- | --- |
| Reactor Core | MANUAL |
| Pressurizer | MANUAL |
| CVCS | MANUAL (this phase) |
| RCPs | MANUAL (this phase) |
| Steam Generators | AUTO |
| Turbine/Generator | AUTO |

# 2. CVCS Model

## 2.1 System Functions

1. Maintain RCS inventory (charging/letdown balance)

2. Control pressurizer level

3. Adjust boron concentration for reactivity control

4. Provide seal injection to RCPs

5. Purify reactor coolant

## 2.2 Flow Paths

LETDOWN PATH:

Hot leg → Regen HX → Letdown orifice → Non-regen HX → Demineralizers → VCT

Normal: 75 gpm, Max: 120 gpm

CHARGING PATH:

VCT → Charging pumps → Regen HX → RCS (cold leg)

Normal: 55 gpm to RCS + 32 gpm to seals = 87 gpm total

## 2.3 State Variables

state.cvcs = {

vctLevel: 72,           // percent (normal 72%)

vctBoron: 1500,         // ppm in VCT

letdownFlow: 75,        // gpm

chargingFlow: 87,       // gpm total

chargingToRCS: 55,      // gpm to RCS

sealInjection: 32,      // gpm to RCP seals (8 per pump)

boricAcidFlow: 0,       // gpm from boric acid tank

dilutionFlow: 0,        // gpm from demineralized water

chargingPumps: [true, false, false], // 3 pumps, normally 1 running

}

## 2.4 Boron Dilution/Boration

Boron changes occur by blending in the VCT:

function updateVCTBoron(state, dt) {

const vctVolume = 1000 * (state.cvcs.vctLevel / 100); // gallons

// Boron entering from letdown (RCS concentration)

const boronIn = state.cvcs.letdownFlow * state.boronConcentration;

// Boron leaving via charging (VCT concentration)

const boronOut = state.cvcs.chargingFlow * state.cvcs.vctBoron;

// Boric acid addition (7000 ppm stock solution)

const boricAcidBoron = state.cvcs.boricAcidFlow * 7000;

// Dilution water (0 ppm)

const dilutionBoron = 0;

// Net boron change in VCT

const netBoronRate = boronIn + boricAcidBoron - boronOut; // ppm·gpm

state.cvcs.vctBoron += (netBoronRate / vctVolume) * (dt / 60);

}

## 2.5 Transport Delay

Critical: Changes in VCT boron take 5-15 minutes to reach the core:

// Model as pipe delay + mixing

const CVCS_TRANSPORT_TIME = 600; // seconds (~10 min)

// Use delay buffer to track boron in transit

state.boronPipeline = [...]; // ring buffer of {time, concentration}

## 2.6 Level Control

function levelController(state) {

const levelError = state.pressurizer.level - levelProgram(state.Tavg);

if (levelError > 3) {

// Level high - increase letdown

state.cvcs.letdownFlow = Math.min(120, 75 + levelError * 5);

} else if (levelError < -3) {

// Level low - increase charging

state.cvcs.chargingToRCS = Math.min(90, 55 - levelError * 5);

} else {

state.cvcs.letdownFlow = 75;

state.cvcs.chargingToRCS = 55;

}

}

# 3. RCP Model

## 3.1 Pump Characteristics

| Parameter | Value |
| --- | --- |
| Number of pumps | 4 (one per loop) |
| Design flow per pump | 97,600 gpm |
| Design head | 280 ft |
| Motor power | ~7,000 HP (5.2 MW) |
| Nominal speed | 1189 rpm |
| Moment of inertia | ~90,000 lb·ft² |
| Coastdown time constant | ~12 seconds |

## 3.2 State Variables

state.rcps = [

{ running: true, speed: 1189, flow: 97600, motorAmps: 350 },

{ running: true, speed: 1189, flow: 97600, motorAmps: 350 },

{ running: true, speed: 1189, flow: 97600, motorAmps: 350 },

{ running: true, speed: 1189, flow: 97600, motorAmps: 350 },

];

## 3.3 Pump Trip and Coastdown

function tripRCP(pumpIndex) {

state.rcps[pumpIndex].running = false;

// Coastdown follows exponential decay

// τ ≈ I·ω / (friction + hydraulic load) ≈ 12 seconds

}

function updateRCPSpeed(pump, dt) {

if (!pump.running) {

// Coastdown: speed decays exponentially

const tau = 12; // seconds

pump.speed *= Math.exp(-dt / tau);

if (pump.speed < 50) pump.speed = 0;

}

// Flow proportional to speed

pump.flow = 97600 * (pump.speed / 1189);

}

## 3.4 Total RCS Flow

function calculateTotalFlow(state) {

let totalFlow = 0;

for (const pump of state.rcps) {

totalFlow += pump.flow;

}

state.flow = totalFlow;

// Update core ΔT based on flow

// ΔT ∝ Power / Flow

const flowFraction = totalFlow / 390400; // nominal

state.deltaT = 61 * state.thermalPower / flowFraction;

}

## 3.5 Heat Addition from RCPs

Each RCP adds ~21 MWt of heat to the coolant (motor + friction):

const RCP_HEAT = 21; // MW per pump

state.rcpHeat = state.rcps.filter(p => p.running).length * RCP_HEAT;

# 4. Reactor Trip Integration

## 4.1 Low Flow Trip

Reactor trips if flow drops below safe limits:

function checkFlowTrip(state) {

const flowPercent = state.flow / 390400 * 100;

// Trip if <87% flow with reactor power >10%

if (flowPercent < 87 && state.neutronPower > 0.10) {

reactorTrip('LOW RCS FLOW');

}

}

## 4.2 RCP Operating Limits

| Condition | Limit | Action |
| --- | --- | --- |
| Minimum RCS pressure | >1500 psig | NPSH protection |
| Minimum RCS temp | >160°F | Bearing protection |
| Maximum ΔT (3 pumps) | <74°F | Thermal limits |
| Maximum ΔT (2 pumps) | <105°F | Thermal limits |

# 5. Operator Interface

## 5.1 CVCS Panel

Displays:

- VCT level (%) with trend

- Letdown flow (gpm)

- Charging flow (gpm)

- RCS boron concentration (ppm) with trend

- VCT boron concentration (ppm)

- Charging pump status (3 pumps)

Controls:

- Letdown flow setpoint (45/60/75/120 gpm)

- Charging pump start/stop

- Boric acid addition rate (gpm)

- Dilution water rate (gpm)

- AUTO/MANUAL mode select

## 5.2 RCP Panel

Per-pump displays:

- Running status (RUN/TRIP/COAST)

- Speed (rpm)

- Motor amps

- Seal injection flow

- Bearing temperatures

Controls:

- START/STOP pushbuttons (with permissives)

## 5.3 New Alarms

| Alarm | Setpoint | Action |
| --- | --- | --- |
| VCT LEVEL HIGH | >85% | Divert letdown |
| VCT LEVEL LOW | <15% | Swap to RWST |
| RCP TRIP | Any pump | Check flow/power |
| LOW RCS FLOW | <87% | Reactor trip |

# 6. Test Scenarios

## 6.1 Boron Dilution

Starting: 50% power, 1200 ppm boron, equilibrium

Action: Initiate dilution at 30 gpm

Expected:

- VCT boron decreases immediately

- RCS boron starts dropping after ~10 minute delay

- Positive reactivity added → power rises if no rod compensation

- Operator must control with rods or stop dilution

## 6.2 Single RCP Trip

Starting: 100% power, 4 RCPs running

Action: Trip one RCP

Expected:

- Flow drops from 100% to ~75% during coastdown

- Core ΔT increases (same power, less flow)

- If power not reduced, potential for DNB

- Runback to 75% power or trip if flow <87%

## 6.3 Loss of All Charging

Starting: Normal operation, 1 charging pump running

Action: Trip charging pump

Expected:

- Pressurizer level drops (letdown continues)

- RCP seal flow from RCS only (no injection)

- Must restore charging or isolate letdown

# 7. Validation Criteria

## 7.1 CVCS

| Test | Expected | Criterion |
| --- | --- | --- |
| Steady state inventory | VCT level stable | ±2%/hour |
| Boron transport delay | 5-15 min lag | Measurable delay |
| 100 ppm dilution | ~900 pcm reactivity | ±100 pcm |

## 7.2 RCPs

| Test | Expected | Criterion |
| --- | --- | --- |
| Coastdown duration | ~30 sec to 25% speed | τ = 12s ± 3s |
| 1 pump trip → flow | 75% flow steady state | ±5% |
| Flow trip setpoint | Trips at <87% flow | Verified |

# 8. Deliverables

systems/cvcs.js - CVCS model with boron tracking

systems/rcp.js - RCP dynamics with coastdown

systems/boron-transport.js - Delay model for boron

ui/cvcs-panel.js - CVCS controls and displays

ui/rcp-panel.js - RCP status and controls

tests/cvcs.test.js - Boron dilution/boration tests

tests/rcp.test.js - Pump trip and coastdown tests

Phase 4 Complete → Proceed to Phase 5 (Steam Generators)