PHASE 5

STEAM GENERATORS

Secondary Side Heat Removal

"Where reactor heat becomes useful work"

Duration: 3-4 weeks

# 1. Phase Overview

Phase 5 replaces the auto steam generators with physics-based models. This closes the thermal-hydraulic loop: reactor heat must now be actively removed by the secondary side, or RCS temperature will rise.

The SGs are the largest heat exchangers in the plant. Their dynamics determine how well the plant can follow load and how it responds to secondary side disturbances.

## 1.1 System Status After Phase 5

| System | Status |
| --- | --- |
| Reactor Core | MANUAL |
| Pressurizer | MANUAL |
| CVCS | MANUAL |
| RCPs | MANUAL |
| Steam Generators (4) | MANUAL (this phase) |
| Turbine/Generator | AUTO |

## 1.2 Critical Coupling

The SGs couple the primary and secondary sides:

- If SGs remove less heat than reactor produces → RCS heats up → pressure rises

- If SGs remove more heat than reactor produces → RCS cools → pressure drops

This creates the fundamental load-following challenge.

# 2. Steam Generator Physics

## 2.1 Design Parameters (Model F)

| Parameter | Value |
| --- | --- |
| Number of SGs | 4 (one per loop) |
| Heat transfer area | ~55,000 ft² per SG |
| U-tube count | ~5,600 tubes per SG |
| Primary flow | 35 × 10⁶ lb/hr per SG |
| Secondary side volume | ~6,000 ft³ per SG |
| Full power steam flow | 3.8 × 10⁶ lb/hr per SG |
| Steam pressure (full power) | 1,000 psia (69 bar) |
| Steam temperature | 545°F (saturation) |
| Feedwater temperature | 440°F |

## 2.2 State Variables (per SG)

state.steamGenerators = [

{

primaryInletTemp: 619,    // °F (from hot leg)

primaryOutletTemp: 558,   // °F (to cold leg)

secondaryPressure: 1000,  // psia

steamTemp: 545,           // °F (saturation)

level: 50,                // percent narrow range

steamFlow: 3.8e6,         // lb/hr

feedwaterFlow: 3.8e6,     // lb/hr

feedwaterTemp: 440,       // °F

heatTransferRate: 853,    // MW (3411/4)

},

// ... 3 more SGs

];

## 2.3 Heat Transfer Model

function sgHeatTransfer(sg, state) {

// Primary side: hot leg temp minus cold leg temp

const T_primary_avg = (sg.primaryInletTemp + sg.primaryOutletTemp) / 2;

// Secondary side: saturation temperature at pressure

const T_secondary = saturationTemperature(sg.secondaryPressure);

// Log mean temperature difference

const LMTD = (sg.primaryInletTemp - T_secondary) - (sg.primaryOutletTemp - T_secondary);

// Q = U × A × LMTD

const UA = 45e6; // BTU/hr·°F (sized for design conditions)

const Q = UA * LMTD / 1e6 * 0.293; // MW

return Q;

}

# 3. Secondary Side Dynamics

## 3.1 Steam Pressure

Steam pressure depends on the balance between steam generation and steam flow out:

function updateSteamPressure(sg, dt) {

// Steam generated from heat transfer

const hfg = latentHeat(sg.secondaryPressure); // BTU/lb

const steamGenerated = sg.heatTransferRate * 3.412e6 / hfg; // lb/hr

// Net mass change in steam dome

const netSteam = steamGenerated - sg.steamFlow; // lb/hr

// Pressure change from mass change

// dP/dt = (net mass rate) / (steam compressibility × volume)

const steamVolume = 3000; // ft³ steam space

const dP = netSteam * dt / 3600 / (steamVolume * steamCompressibility(sg.secondaryPressure));

sg.secondaryPressure += dP;

sg.steamTemp = saturationTemperature(sg.secondaryPressure);

}

## 3.2 Level Control

SG level is counterintuitive: 'shrink and swell' causes level to initially move opposite to expectation.

// Shrink: Power increases → more boiling → steam volume increases

//         → liquid level DROPS even though inventory unchanged

// Swell: Power decreases → less boiling → steam volume decreases

//        → liquid level RISES even though inventory unchanged

function updateSGLevel(sg, dt) {

// Mass balance: feedwater in, steam out

const netMass = (sg.feedwaterFlow - sg.steamFlow) * dt / 3600; // lb

// Void fraction effect (shrink/swell)

const voidFraction = sg.heatTransferRate / 853 * 0.3; // 0-30% void at power

const apparentLevel = sg.massInventory / DESIGN_INVENTORY * (1 - voidFraction);

sg.level = apparentLevel * 100; // percent NR

}

## 3.3 Feedwater Control

function feedwaterController(sg) {

// Three-element control: level, steam flow, feedwater flow

const levelError = sg.level - 50; // 50% setpoint

const flowMismatch = sg.steamFlow - sg.feedwaterFlow;

// Adjust feedwater to match steam plus level correction

sg.feedwaterFlow = sg.steamFlow + levelError * 50000 + flowMismatch * 0.5;

// Clamp to physical limits

sg.feedwaterFlow = Math.max(0, Math.min(5e6, sg.feedwaterFlow));

}

# 4. Primary-Secondary Coupling

## 4.1 Heat Balance

At steady state: Q_reactor = Q_SGs

During transients, the mismatch causes RCS temperature to change:

function updateRCSTemperatures(state, dt) {

// Heat added by reactor

const Q_reactor = state.thermalPower * 3411; // MW

// Heat removed by SGs

let Q_SGs = 0;

for (const sg of state.steamGenerators) {

Q_SGs += sg.heatTransferRate;

}

// Heat added by RCPs

const Q_RCPs = state.rcps.filter(p => p.running).length * 21; // MW

// Net heat into RCS

const Q_net = Q_reactor + Q_RCPs - Q_SGs; // MW

// Temperature change

const dT = Q_net * dt / THERMAL_MASS * 3412 / 3600; // °F

state.Tavg += dT;

}

## 4.2 Cold Leg Temperature Update

// Cold leg temp = SG outlet temp (primary side)

function updateColdLeg(state) {

let totalColdLegFlow = 0;

let weightedTemp = 0;

for (let i = 0; i < 4; i++) {

const sg = state.steamGenerators[i];

const rcp = state.rcps[i];

weightedTemp += sg.primaryOutletTemp * rcp.flow;

totalColdLegFlow += rcp.flow;

}

state.Tcold = weightedTemp / totalColdLegFlow;

state.Thot = state.Tcold + state.deltaT;

state.Tavg = (state.Thot + state.Tcold) / 2;

}

# 5. Steam Dump System

## 5.1 Functions

The steam dump system bypasses steam to the condenser when turbine demand is less than reactor power:

- After reactor trip: removes decay heat

- During load rejection: absorbs excess steam

- During cooldown: controlled heat removal

## 5.2 Control Modes

state.steamDump = {

mode: 'Tavg',         // 'Tavg' or 'PRESSURE'

valvePosition: 0,      // 0-100%

capacity: 40,          // % of full steam flow

};

// Tavg mode: modulate to maintain Tavg setpoint

function steamDumpTavgMode(state) {

const error = state.Tavg - tavgProgram(state.loadDemand);

if (error > 5) {

// Tavg too high, dump steam

state.steamDump.valvePosition = Math.min(100, error * 10);

} else {

state.steamDump.valvePosition = 0;

}

}

// Pressure mode: modulate to maintain steam pressure

function steamDumpPressureMode(state) {

const avgPressure = state.steamGenerators.reduce((s,sg) => s + sg.secondaryPressure, 0) / 4;

if (avgPressure > 1050) {

state.steamDump.valvePosition = Math.min(100, (avgPressure - 1050) * 5);

} else {

state.steamDump.valvePosition = 0;

}

}

# 6. Operator Interface

## 6.1 SG Panel (per SG)

Displays:

- Steam pressure (psia) with trend

- Narrow range level (%) with trend

- Wide range level (%)

- Steam flow (Mlb/hr)

- Feedwater flow (Mlb/hr)

- Feedwater temperature (°F)

- Primary inlet/outlet temps (°F)

Controls:

- Feedwater control: AUTO/MANUAL

- Feedwater flow setpoint (manual mode)

- Main steam isolation valve (OPEN/CLOSE)

## 6.2 Steam Dump Panel

Displays:

- Mode (Tavg/PRESSURE)

- Valve position (%)

- Dump flow (lb/hr)

Controls:

- Mode select

- ARM/DISARM

## 6.3 Alarms

| Alarm | Setpoint | Action |
| --- | --- | --- |
| SG LEVEL HIGH-HIGH | >80% NR | Trip turbine |
| SG LEVEL LOW-LOW | <15% NR | Trip reactor |
| STEAM PRESS HIGH | >1100 psia | Safety valve lift |
| SG ΔP HIGH | >50 psi between SGs | Check MSIV |

# 7. Test Scenarios

## 7.1 Load Rejection

Starting: 100% power, turbine at 100% load

Action: Turbine trips (load rejection to house loads only)

Expected:

- Steam flow drops to near zero

- Steam pressure rises rapidly

- Steam dumps open (Tavg mode)

- SG level swells, then recovers

- Reactor power may runback or trip depending on response

## 7.2 Loss of Feedwater to One SG

Starting: 100% power, normal feedwater

Action: Lose feedwater to SG #1

Expected:

- SG #1 level drops, others compensate partially

- Heat removal imbalance → affected loop hotter

- If level reaches LO-LO, reactor trips

- Other SGs can remove ~75% of heat

## 7.3 Cooldown Using Steam Dump

Starting: Post-trip, reactor subcritical, SGs available

Action: Initiate controlled cooldown at 50°F/hr

Expected:

- Steam dump in pressure mode, gradually lowered

- RCS cools at controlled rate

- Pressurizer level drops with contraction

- Must not exceed P-T limits

# 8. Validation Criteria

## 8.1 Steady State

| Parameter | Expected | Tolerance |
| --- | --- | --- |
| Steam pressure at 100% | 1000 psia | ±20 psia |
| SG level at 100% | 50% NR | ±5% |
| Tcold at 100% | 558°F | ±3°F |
| Heat transfer (total) | 3411 MW | ±50 MW |

## 8.2 Transient Response

| Transient | Expected | Criterion |
| --- | --- | --- |
| 10% load increase | Pressure dip ~30 psi | Recovery <60s |
| Turbine trip | Pressure rise to ~1050 | No safety lift |
| Shrink on power increase | Level drops 5-10% | Recovers in 2-3 min |

# 9. Deliverables

systems/steam-generator.js - SG heat transfer and dynamics

systems/feedwater-control.js - Three-element controller

systems/steam-dump.js - Steam dump system

ui/sg-panel.js - Per-SG displays and controls

ui/steam-dump-panel.js - Steam dump controls

tests/sg-dynamics.test.js - Heat transfer validation

tests/shrink-swell.test.js - Level response validation

Phase 5 Complete → Proceed to Phase 6 (Turbine/Generator)