PHASE 6

TURBINE/GENERATOR

Complete System Integration

"The final piece: real load demand and grid interaction"

Duration: 2-3 weeks

# 1. Phase Overview

Phase 6 completes the plant by adding the turbine-generator system. After this phase, ALL systems are manual - the operator controls the entire power conversion chain from reactor to grid.

The turbine creates the demand signal that drives the entire plant. Load changes propagate backward: turbine demands steam → SGs must provide steam → reactor must provide heat.

## 1.1 Final System Status

| System | Status |
| --- | --- |
| Reactor Core | MANUAL |
| Pressurizer | MANUAL |
| CVCS | MANUAL |
| RCPs (4) | MANUAL |
| Steam Generators (4) | MANUAL |
| Turbine/Generator | MANUAL (this phase) |

## 1.2 Complete Energy Flow

With all systems manual, energy flows through the complete chain:

Reactor (fission) → Coolant → SGs → Steam → Turbine → Generator → Grid

Each link must be balanced. Imbalances cause temperature, pressure, or frequency deviations.

# 2. Turbine Model

## 2.1 Design Parameters

| Parameter | Value |
| --- | --- |
| Type | Tandem-compound, 4-flow LP |
| Rated output | ~1150 MW |
| Rated speed | 1800 rpm (60 Hz, 4-pole) |
| Inlet steam pressure | 1000 psia |
| Inlet steam quality | 99.75% (nearly saturated) |
| Steam flow (rated) | 15.2 × 10⁶ lb/hr |
| Exhaust pressure | ~1-2 psia (condenser) |
| Efficiency | ~34% (typical for saturated cycle) |

## 2.2 State Variables

state.turbine = {

speed: 1800,           // rpm

power: 0,              // MW

steamFlow: 0,          // lb/hr

throttlePosition: 0,   // 0-100%

governorPosition: 0,   // 0-100%

status: 'OFFLINE',     // OFFLINE/WARMING/ROLLING/SYNCED/TRIPPED

loadSetpoint: 0,       // MW

};

state.generator = {

mwOutput: 0,           // MW electrical

mvarOutput: 0,         // MVAR reactive

voltage: 0,            // kV

current: 0,            // kA

frequency: 60,         // Hz

breaker: 'OPEN',       // OPEN/CLOSED

};

## 2.3 Turbine Control Valves

Steam admission is controlled by governor valves:

function turbineValveControl(state) {

// Governor valve position sets steam flow

// Flow is roughly linear with valve position

const maxFlow = 15.2e6; // lb/hr at 100% valve

state.turbine.steamFlow = maxFlow * (state.turbine.governorPosition / 100);

// This steam flow is the DEMAND on the SGs

// SGs must supply this or pressure will change

for (const sg of state.steamGenerators) {

sg.steamFlow = state.turbine.steamFlow / 4;

}

}

# 3. Generator Model

## 3.1 Power Conversion

function updateGenerator(state, dt) {

// Turbine mechanical power → generator electrical power

// Efficiency ~98-99% for large generators

const efficiency = 0.985;

state.generator.mwOutput = state.turbine.power * efficiency;

// Frequency depends on speed

// f = (rpm × poles) / 120

// For 4-pole: f = rpm / 30

state.generator.frequency = state.turbine.speed / 30;

}

## 3.2 Grid Synchronization

Before connecting to grid, generator must match:

- Frequency: Within 0.05 Hz of grid (60 Hz)

- Voltage: Within 5% of grid voltage

- Phase angle: Within 10 degrees

function checkSyncConditions(state) {

const freqOK = Math.abs(state.generator.frequency - 60) < 0.05;

const voltOK = Math.abs(state.generator.voltage - 22) < 1; // 22 kV typical

const phaseOK = Math.abs(state.generator.phaseAngle) < 10;

return freqOK && voltOK && phaseOK;

}

## 3.3 Load Control

Once synchronized, governor controls load:

function governorControl(state, dt) {

if (state.generator.breaker === 'OPEN') {

// Not synced: control speed

const speedError = 1800 - state.turbine.speed;

state.turbine.governorPosition += speedError * 0.01 * dt;

} else {

// Synced: control load

const loadError = state.turbine.loadSetpoint - state.generator.mwOutput;

state.turbine.governorPosition += loadError * 0.001 * dt;

}

state.turbine.governorPosition = clamp(state.turbine.governorPosition, 0, 100);

}

# 4. Turbine Speed/Load Control

## 4.1 Operating Modes

| Mode | Use | Control |
| --- | --- | --- |
| Speed Control | Before sync | Maintain 1800 rpm |
| Droop | Grid-connected | 4-5% droop characteristic |
| Base Load | Maximum output | Hold valves at limit |
| Load Following | Variable load | Follow setpoint |

## 4.2 Droop Control

Droop allows load sharing with other generators:

// 5% droop means: 5% speed drop = 100% load increase

// At 60 Hz, full load: 60 Hz

// At 60 Hz, no load: 63 Hz (100% × 5% × 60 = 3 Hz higher)

function droopController(state) {

const droopPercent = 5;

const speedSetpoint = 1800 * (1 + droopPercent/100 * (1 - state.turbine.loadSetpoint/1150));

const speedError = speedSetpoint - state.turbine.speed;

return speedError * GOVERNOR_GAIN;

}

## 4.3 Load Ramp Rates

| Condition | Max Rate |
| --- | --- |
| Normal load changes | 5%/min (57.5 MW/min) |
| Step load change | ±10% (115 MW) |
| Load rejection capacity | 50% without trip |
| Minimum load | ~25% (for stability) |

# 5. Complete Plant Integration

## 5.1 Load-Following Dynamics

When operator increases load setpoint:

1. Governor opens valves → more steam to turbine

2. Steam pressure drops (demand > supply)

3. SG heat transfer increases (larger ΔT)

4. Tcold drops, then Thot drops

5. Tavg drops → positive MTC feedback → power rises

6. Or: Rod control system withdraws rods to raise Tavg

7. Reactor power increases to match new steam demand

8. New equilibrium at higher load

## 5.2 Master Simulation Loop

function masterSimulationStep(dt) {

// 1. Turbine/Generator (sets steam demand)

governorControl(state, dt);

turbineValveControl(state);

updateTurbinePower(state, dt);

updateGenerator(state, dt);

// 2. Steam Generators (respond to steam demand)

for (const sg of state.steamGenerators) {

updateSteamPressure(sg, dt);

sgHeatTransfer(sg, state);

feedwaterController(sg);

updateSGLevel(sg, dt);

}

// 3. RCS Temperatures (respond to SG heat removal)

updateRCSTemperatures(state, dt);

updateColdLeg(state);

// 4. Reactor (responds to temperature feedback)

const rho = calculateReactivity(state);

state.neutronPower = solveKinetics(state.neutronPower, rho, dt);

state.thermalPower = thermalLag(state.neutronPower, state.thermalPower, dt);

state.fuelTemp = state.Tavg + 400 * state.thermalPower;

// 5. Pressurizer (responds to thermal expansion)

const surgeFlow = calculateSurgeFlow(state, dt);

updatePressurizerVolumes(state, surgeFlow, dt);

updatePressurizerPressure(state, dt);

pressureController(state, dt);

// 6. CVCS (maintains inventory and boron)

levelController(state);

updateVCTBoron(state, dt);

updateRCSBoron(state, dt);

// 7. RCPs (provide flow)

for (const pump of state.rcps) {

updateRCPSpeed(pump, dt);

}

calculateTotalFlow(state);

// 8. Protection System Checks

checkAllTripConditions(state);

}

# 6. Operator Interface

## 6.1 Turbine Panel

Displays:

- Turbine speed (rpm)

- Generator output (MW, MVAR)

- Frequency (Hz)

- Steam flow (Mlb/hr)

- Governor valve position (%)

- First stage pressure (psia)

Controls:

- Turbine START/STOP

- Speed/Load setpoint

- Mode select (SPEED/LOAD)

- Generator breaker CLOSE/TRIP

## 6.2 Main Control Board Overview

With all systems manual, the main control board shows:

┌─────────────────────────────────────────────────────────────┐

│  REACTOR      PRESSURIZER     TURBINE/GEN                  │

│  Power: 100%  Press: 2235     Load: 1150 MW                │

│  Rods: 200    Level: 60%      Freq: 60.00 Hz               │

│  Tavg: 588°F  Htrs: AUTO      Speed: 1800 rpm              │

│                                                             │

│  RCPs         CVCS            STEAM GENERATORS             │

│  1:RUN 2:RUN  VCT: 72%        #1: 1000/50%  #3: 1000/50%   │

│  3:RUN 4:RUN  Boron: 800      #2: 1000/50%  #4: 1000/50%   │

│  Flow: 100%   Chg: 87 gpm     Steam: 15.2 Mlb/hr           │

└─────────────────────────────────────────────────────────────┘

# 7. Test Scenarios

## 7.1 Complete Startup

Cold shutdown to 100% power:

1. Energize RCPs, establish flow

2. Heatup RCS using RCP heat

3. Establish pressurizer steam bubble

4. Take reactor critical, raise to ~5% power

5. Roll turbine, synchronize to grid

6. Raise power to 100% at 5%/min

## 7.2 Load Follow

Starting: 100% power, synchronized

Action: Reduce load to 50%, hold 4 hours, return to 100%

Expected:

- Steam demand drops → Tavg rises initially

- Rods insert (or boron added) to control Tavg

- Xenon builds at reduced power

- When returning to 100%, must overcome xenon

- May require maximum rod withdrawal + dilution

## 7.3 Generator Trip (Load Rejection)

Starting: 100% power, synchronized

Action: Generator breaker opens (loss of load)

Expected:

- Turbine speed rises (no load)

- Steam demand drops to zero

- Steam pressure rises rapidly

- Reactor may trip on high Tavg or turbine trip

- Steam dumps open to remove heat

## 7.4 Turbine Trip

Starting: 100% power

Action: Turbine trips (stop valves close)

Expected:

- All steam flow stops instantly

- Steam pressure rises to safety valve setpoint

- Reactor trips on turbine trip signal

- Steam dumps handle decay heat

# 8. Final Validation

## 8.1 Complete System Tests

| Scenario | Success Criteria |
| --- | --- |
| Cold startup to 100% power | Reaches full power without trips |
| 50% load reduction and return | Completes cycle, manages xenon |
| Reactor trip from 100% | All systems respond correctly |
| Single component failures | Operators can stabilize plant |

## 8.2 Steady State (100% Power)

| Parameter | Value | Tolerance |
| --- | --- | --- |
| Reactor thermal power | 3411 MW | ±20 MW |
| Generator output | ~1150 MW | ±20 MW |
| Tavg | 588°F | ±2°F |
| RCS pressure | 2235 psig | ±10 psig |
| Steam pressure | 1000 psia | ±20 psia |
| Grid frequency | 60.00 Hz | ±0.02 Hz |

# 9. Deliverables

## 9.1 Phase 6 Code

systems/turbine.js - Turbine model with valves

systems/generator.js - Generator and grid sync

systems/governor.js - Speed/load control

ui/turbine-panel.js - Turbine controls

ui/main-control-board.js - Integrated overview

## 9.2 Integration Code

simulation/master-loop.js - Complete plant simulation

simulation/protection-system.js - All trip logic

simulation/state-manager.js - Unified state management

## 9.3 Final Documentation

OPERATING_GUIDE.md - How to operate the simulator

SYSTEM_DESCRIPTIONS.md - Technical documentation

SCENARIO_GUIDE.md - Training scenarios

# 10. Project Complete

Upon completion of Phase 6, the PWR simulator has:

✓ Validated physics engine with correct thermodynamic properties

✓ Reactor core with point kinetics and reactivity feedback

✓ Pressurizer with steam compression and heater/spray control

✓ CVCS with boron transport delay and inventory control

✓ Four RCPs with coastdown dynamics

✓ Four steam generators with shrink/swell behavior

✓ Turbine-generator with governor control and grid sync

✓ Complete protection system with realistic trip setpoints

✓ Integrated control board for full plant operation

The simulator provides a realistic, challenging experience for understanding PWR operations.