WESTINGHOUSE 4-LOOP PWR

CONTROL ROOM INDICATOR TYPES

Blender 5.0 → Unity 6.3 Construction Manual

Companion Volume to the Analog Round Gauge Manual

Vertical Edgewise Meters • Vertical Bar Graphs • Strip Chart Recorders

Annunciator Windows • Indicator Lamps • Digital Numeric Readouts

Nuclear Reactor Simulator Project — February 2026

# Table of Contents

Update this Table of Contents in Word/LibreOffice by right-clicking and selecting Update Field.

# Part 1: Real-World PWR Control Room Indicator Types

A Westinghouse 4-Loop PWR main control room employs a diverse array of instrumentation displays across its vertical panels and benchboards. The control room is arranged in a horseshoe or wrap-around configuration, divided into functional sections: the reactor/RCS panel, steam generator and feedwater panel, turbine-generator panel, electrical distribution panel, and engineered safety features panel. Each section uses a specific mix of indicator types optimized for the information being conveyed.

The companion Analog Round Gauge Manual covered the classic Bourdon-tube style circular dial gauge in complete detail. This manual covers the six remaining major indicator types found on Westinghouse PWR control panels, each serving a distinct operational purpose within the control room human-system interface.

## 1.1 Complete Indicator Type Inventory

The following table catalogues every major indicator type found on a real Westinghouse 4-Loop PWR main control room panel, with typical parameters and panel locations.

| Indicator Type | Typical Use | Panel Location |
| --- | --- | --- |
| Analog Round Gauge | Temperatures, pressures, levels | Vertical panel faces, eye level |
| Vertical Edgewise Meter | Loop temps, SG levels, motor currents, flows | Dense vertical arrays on panel faces |
| Vertical Bar Graph Display | PZR level, SG levels, flow rates | Modernized panel sections |
| Strip Chart Recorder | Temperature/pressure/level/power trends | Benchboards and vertical faces |
| Annunciator Window Tile | All alarm conditions | Grid arrays at TOP of all panels |
| Indicator Lamp (Status Light) | Pump/valve/breaker component status | Embedded in mimic bus diagrams |
| Digital Numeric Readout | Rod position, power %, precise readings | Where high precision is needed |

## 1.2 Design Philosophy

Each indicator type is modeled as a 2.5D asset in Blender 5.0, exported as FBX, imported into Unity 6.3, and rendered to a RenderTexture displayed on the GUI panel via a RawImage component. This provides photorealistic fidelity while maintaining excellent runtime performance. All dimensions, color schemes, scale markings, and behavioral characteristics are based on real Westinghouse 4-Loop PWR instrumentation as documented in NRC NUREG-0700, Weschler Instruments nuclear-grade specifications, and standard nuclear industry human factors engineering practice.