# Part 2: Vertical Edgewise Meter

## 2.1 Real-World Reference

The vertical edgewise meter is the single most common indicator on a Westinghouse PWR control panel. The industry standard is the Weschler VX-252 and VC-252 series, designed and seismically qualified specifically for the nuclear power industry per IEEE Standard 344-1987 and IEEE Standard 420-1973. These meters occupy a narrow vertical profile approximately 2.5 inches wide by 6 inches tall, allowing operators to compare many parameters at a glance in dense side-by-side arrays.

A thin knife-edge pointer moves vertically along a printed linear scale on the left or right side of the face. The faceplate is white or off-white with black scale markings and a black pointer. On a Westinghouse 4-Loop PWR, edgewise meters monitor: RCS loop temperatures (T-hot and T-cold for all 4 loops), SG narrow-range levels, RCP motor currents, charging and letdown flow, main steam pressures, and containment sump levels.

## 2.2 Specifications

| Property | Specification |
| --- | --- |
| Overall Dimensions | 2.5" W × 6.0" H × 2.5" D (front face visible) |
| Faceplate | White/off-white background, matte finish |
| Scale | Vertical, linear, printed on left or right side |
| Major Graduations | Every 50 or 100 units, with numeric labels |
| Minor Graduations | Every 10 or 25 units |
| Pointer | Black knife-edge, translates vertically |
| Pointer Travel | Approximately 4.5" of vertical travel |
| Bezel | Black plastic rectangular frame |
| Accuracy | 2% per ANSI C39.1, calibratable to 1% |
| Seismic Qualification | IEEE 344-1987 |

## 2.3 Parameter Ranges

| Parameter | Edgewise Meter Range |
| --- | --- |
| T-hot / T-cold (per loop) | 100–650°F |
| SG Narrow Range Level | 0–100% |
| RCP Motor Current | 0–500 A |
| Charging Flow | 0–150 gpm |
| Letdown Flow | 0–120 gpm |
| Main Steam Pressure | 0–1200 psig |

## 2.4 Blender 5.0 Modeling

### Scene Setup

- Open Blender 5.0, delete default cube. Set units: Metric, Unit Scale 0.01 (1 BU = 1 cm).

- Set viewport background to neutral dark gray (#1A1A1A).

### Meter Body

- Add Cube. Dimensions: X=6.35cm, Y=0.5cm, Z=15.24cm. Name "EdgewiseBody".

- Edit Mode: select front (+Y) face, Inset (I) by 0.2cm for bezel border, extrude inner face back -0.1cm for recess.

- Apply black plastic material: Base Color #1A1A1A, Roughness 0.7, Metallic 0.0.

### Faceplate

- Add Plane, rotate R X 90. Dimensions: X=5.95cm, Z=14.64cm. Position Y=0.21cm. Name "EdgewiseFace".

- UV Unwrap (U → Unwrap). Apply material with Image Texture for the scale.

TIP: Create scale texture in GIMP/Photoshop: 256×512px, white background, vertical scale line on left with major ticks every ~50px, minor ticks every ~10px. Black numerals in small sans-serif font. Export as PNG.

### Pointer

- Add Plane. Reshape in Edit Mode to a thin horizontal knife: ~3.0cm wide (X) × 0.15cm tall (Z). Merge two left vertices to form a pointed tip.

- Position at Y=0.25cm (in front of faceplate). Set origin to bottom-center of the pointer (this is the slide axis anchor).

- Name "EdgewisePointer". Apply solid black material (#0A0A0A, Roughness 0.3).

WARNING: Unlike the round gauge needle which rotates, the edgewise pointer translates vertically. The C# script moves this object along local Z. Ensure the origin is correct.

### Camera and Lighting

- Add Camera: Orthographic, position (0, 15, 7.5), rotation (90, 0, 0), Ortho Scale 18.

- Add Area Light above-front: position (0, 10, 20), 50W, 5000K. Optional fill light from below at 10W.

## 2.5 FBX Export

Select all objects. File → Export → FBX: Scale=0.01, Apply Scalings=FBX All, Forward=-Z, Up=Y, Apply Modifiers=checked. Save as EdgewiseMeter.fbx.

## 2.6 Unity 6.3 Setup

- Import FBX to Assets/Models/Gauges/Edgewise/. Scale Factor=1.0. Import textures to Assets/Textures/Gauges/.

- Create layer "GaugeEdgewise". Assign meter prefab to this layer.

- Create camera "EdgewiseCam": Orthographic, Culling Mask=GaugeEdgewise only.

- Create RenderTexture 256×512px. Assign to EdgewiseCam Target Texture.

- Add RawImage on GUI Canvas, assign the RenderTexture.

## 2.7 C# Driver Script

Attach to EdgewisePointer:

using UnityEngine;

public class EdgewiseMeterDriver : MonoBehaviour

{

[Header("Scale Configuration")]

public float scaleMin = 100f;

public float scaleMax = 650f;

[Header("Pointer Travel (local Z positions)")]

public float pointerZMin = 0.015f;  // Bottom of scale

public float pointerZMax = 0.130f;  // Top of scale

[Header("Dynamics")]

public float damping = 8.0f;

private float _targetZ;

public void SetValue(float value)

{

float v = Mathf.Clamp(value, scaleMin, scaleMax);

float t = Mathf.InverseLerp(scaleMin, scaleMax, v);

_targetZ = Mathf.Lerp(pointerZMin, pointerZMax, t);

}

void Update()

{

Vector3 pos = transform.localPosition;

pos.z = Mathf.Lerp(pos.z, _targetZ, Time.deltaTime * damping);

transform.localPosition = pos;

}

}