# Part 8: Universal Integration Guidelines

## 8.1 Standard FBX Export Settings

| Setting | Value |
| --- | --- |
| Scale | 0.01 |
| Apply Scalings | FBX All |
| Forward Axis | -Z Forward |
| Up Axis | Y Up |
| Apply Modifiers | Checked |
| Smoothing | Face |

## 8.2 Unity Layer Architecture

| Layer | Purpose |
| --- | --- |
| GaugeAnalog | Analog round gauges (companion manual) |
| GaugeEdgewise | Vertical edgewise meters |
| GaugeBarGraph | Vertical bar graph displays |
| GaugeStripChart | Strip chart recorders |
| GaugeAnnunciator | Annunciator tile arrays |
| GaugeStatusLamp | Indicator lamp / mimic panels |
| GaugeDigital | Digital numeric readouts |

Each layer has a dedicated Orthographic camera rendering only that layer to its own RenderTexture. Clear Flags = Solid Color (black). Depth value must not conflict with main camera.

## 8.3 RenderTexture Sizing

| Gauge Type | Recommended RT Size |
| --- | --- |
| Analog Round Gauge | 512 × 512 px |
| Vertical Edgewise Meter | 256 × 512 px |
| Vertical Bar Graph | 256 × 512 px |
| Strip Chart Recorder | 512 × 512 px |
| Annunciator Grid (per row of 8) | 1024 × 128 px |
| Indicator Lamp (single) | 128 × 128 px |
| Indicator Lamp (mimic panel) | 1024 × 512 px |
| Digital Readout | 256 × 128 px |

## 8.4 Performance Optimization

- Group gauges that share one RenderTexture where possible (e.g., 4 edgewise meters in a bank).

- Use MaterialPropertyBlock instead of unique Material instances per gauge.

- Render gauge cameras on-demand (camera.Render()) for slow-changing values; every frame for strip charts.

- Use GPU instancing on annunciator tile mesh renderers.

## 8.5 Lighting Standard

- Primary: Area Light, 50W, 4000–5000K warm white, above-front.

- Fill: Point/Area Light, 10–15W, below-front to soften bezel shadows.

- Ambient: Low warm gray (#2A2520) simulating control room wall reflections.

- No directional light (avoid harsh shadows on panel instruments).

## 8.6 NUREG-0700 Color Standards

| Color | Meaning |
| --- | --- |
| Red | Danger, abnormal, trip, alarm, safety concern |
| Green | Normal, safe, running, open |
| Amber/Yellow | Caution, warning, intermediate state |
| White | Neutral status, information only |
| Blue | Bypass, suppressed, maintenance |

# Part 9: Quick Reference and Checklists

## 9.1 Parameter-to-Gauge Selection Guide

| Parameter | Recommended Indicator(s) |
| --- | --- |
| Temperatures (T-hot/T-cold/T-avg) | Analog round gauge OR edgewise + strip chart |
| Pressurizer Pressure | Analog round + digital readout + strip chart |
| Pressurizer Level | Bar graph + strip chart |
| SG Water Levels | Bar graph + edgewise backup + strip chart |
| Reactor Power (%RTP) | Digital readout + analog round gauge |
| Rod Position (steps) | Digital readout (dedicated) |
| Flow Rates (RCS, CVCS) | Edgewise meter OR bar graph |
| Component Status | Indicator lamp (binary on/off) |
| Alarms | Annunciator window tile |
| Nuclear Instrumentation | Edgewise meter (log scale) + strip chart |

## 9.2 File Naming Convention

| Asset Type | Pattern |
| --- | --- |
| Blender source | GaugeType_ParamName.blend |
| FBX export | GaugeType_ParamName.fbx |
| Scale texture | GaugeType_Scale_Range.png |
| Alarm text texture | Annun_System_AlarmText.png |
| Unity prefab | GaugeType_ParamName.prefab |
| C# driver | GaugeTypeDriver.cs |
| RenderTexture | RT_GaugeType_ParamName |

## 9.3 Implementation Checklist

- Identify real-world gauge type and parameter range from this manual.

- Model in Blender 5.0 per type-specific instructions.

- Apply materials with Principled BSDF and emission nodes where needed.

- Set up orthographic camera and standard lighting in Blender.

- Export FBX with standardized settings (Section 8.1).

- Import into Unity 6.3 with correct settings.

- Assign to dedicated layer.

- Create dedicated orthographic camera for that layer.

- Create RenderTexture at correct size (Section 8.3).

- Assign RenderTexture to camera Target Texture.

- Attach C# driver script to animated element.

- Wire SetValue() to the simulation physics engine.

- Add RawImage on GUI canvas, assign RenderTexture.

- Test full parameter range for correct behavior.

- Verify visual consistency with other panel gauges.

--- End of Manual ---